MicroPython

http://micropython.org/

MicroPython is a lean and efficient implementation of the Python 3 programming language that includes a small subset of the Python standard library and is optimised to run on microcontrollers and in constrained environments.

The MicroPython pyboard is a compact electronic circuit board that runs MicroPython on the bare metal, giving you a low-level Python operating system that can be used to control all kinds of electronic projects.

MicroPython is packed full of advanced features such as an interactive prompt, arbitrary precision integers, closures, list comprehension, generators, exception handling and more. Yet it is compact enough to fit and run within just 256k of code space and 16k of RAM.

MicroPython aims to be as compatible with normal Python as possible to allow you to transfer code with ease from the desktop to a microcontroller or embedded system.

The Espressif ESP32 Development Board (image attribution: Adafruit).

목차

MicroPython	1

MicroPython license information	8

Quick reference for the ESP32	12

Installing MicroPython	12

General board control	12

Networking	13

Delay and timing	14

Timers	14

Pins and GPIO	14

PWM (pulse width modulation)	15

ADC (analog to digital conversion)	15

Software SPI bus	16

Hardware SPI bus	17

I2C bus	17

Real time clock (RTC)	18

Deep-sleep mode	18

RMT	18

OneWire driver	19

NeoPixel driver	19

Capacitive touch	19

DHT driver	20

WebREPL (web browser interactive prompt)	20

MicroPython language and implementation	22

Glossary	23

The MicroPython Interactive Interpreter Mode (aka REPL)	25

Auto-indent	25

Auto-completion	25

Interrupting a running program	26

Paste mode	26

Soft reset	26

The special variable _ (underscore)	27

Raw mode	27

MicroPython .mpy files	28

Versioning and compatibility of .mpy files	28

Binary encoding of .mpy files	30

The header	30

Raw code elements	30

Writing interrupt handlers	32

Tips and recommended practices	32

MicroPython issues	32

The emergency exception buffer	32

Simplicity	32

Communication between an ISR and the main program	32

The use of object methods as callbacks	33

Creation of Python objects	33

Use of Python objects	34

Overcoming the float limitation	34

Using micropython.schedule	34

Exceptions	35

General issues	35

Interrupt handler design	35

Reentrancy	36

Critical sections	36

Interrupts and the REPL	37

Maximising MicroPython speed	39

Designing for speed	39

Algorithms	39

RAM allocation	40

Buffers	40

Floating point	40

Arrays	40

Identifying the slowest section of code	41

MicroPython code improvements	41

The const() declaration	41

Caching object references	41

Controlling garbage collection	41

The Native code emitter	41

The Viper code emitter	42

Accessing hardware directly	43

MicroPython on microcontrollers	45

Flash memory	45

RAM	45

Compilation phase	45

Execution phase	46

The heap	48

Fragmentation	49

Reporting	49

Control of garbage collection	50

String operations	50

Postscript	51

Footnote: gc.collect() return value	51

Distribution packages, package management, and deploying applications	52

Overview	52

Distribution packages	52

upip package manager	53

Cross-installing packages	54

Cross-installing packages with freezing	54

Creating distribution packages	55

Application resources	56

References	57

Inline assembler for Thumb2 architectures	58

Document conventions	58

Instruction categories	58

Usage examples	59

References	59

Working with filesystems	60

VFS	60

Block devices	61

Built-in block devices	61

Custom block devices	61

Filesystems	63

FAT	63

Littlefs	64

Hybrid (STM32)	64

Hybrid (ESP32)	65

The pyboard.py tool	66

Running a command on the device	66

Running a script on the device	66

Filesystem access	67

Using the pyboard library	67

MicroPython libraries	69

Python standard libraries and micro-libraries	69

MicroPython-specific libraries	70

Port-specific libraries	70

Libraries specific to the pyboard	71

Libraries specific to the WiPy	71

Libraries specific to the ESP8266 and ESP32	71

MicroPython differences from CPython	73

Syntax	74

Spaces	74

uPy requires spaces between literal numbers and keywords, CPy doesn’t	74

Unicode	74

Unicode name escapes are not implemented	74

Core language	75

Classes	75

Special method __del__ not implemented for user-defined classes	75

Method Resolution Order (MRO) is not compliant with CPython	75

When inheriting from multiple classes super() only calls one class	75

Calling super() getter property in subclass will return a property object, not the value	76

Functions	76

Error messages for methods may display unexpected argument counts	76

User-defined attributes for functions are not supported	76

Generator	77

Context manager __exit__() not called in a generator which does not run to completion	77

Runtime	77

Local variables aren’t included in locals() result	77

Code running in eval() function doesn’t have access to local variables	77

import	78

__path__ attribute of a package has a different type (single string instead of list of strings) in MicroPython	78

Failed to load modules are still registered as loaded	78

MicroPython does’t support namespace packages split across filesystem.	78

Builtin types	80

Exception	80

Exception chaining not implemented	80

User-defined attributes for builtin exceptions are not supported	80

Exception in while loop condition may have unexpected line number	80

Exception.__init__ method does not exist.	80

bytearray	81

Array slice assignment with unsupported RHS	81

bytes	81

bytes objects support .format() method	81

bytes() with keywords not implemented	81

Bytes subscription with step != 1 not implemented	82

dict	82

Dictionary keys view does not behave as a set.	82

float	82

uPy and CPython outputs formats may differ	82

int	82

No int conversion for int-derived types available	82

list	83

List delete with step != 1 not implemented	83

List slice-store with non-iterable on RHS is not implemented	83

List store with step != 1 not implemented	83

str	83

Start/end indices such as str.endswith(s, start) not implemented	83

Attributes/subscr not implemented	84

str(...) with keywords not implemented	84

str.ljust() and str.rjust() not implemented	84

None as first argument for rsplit such as str.rsplit(None, n) not implemented	84

Instance of a subclass of str cannot be compared for equality with an instance of a str	84

Subscript with step != 1 is not yet implemented	85

tuple	85

Tuple load with step != 1 not implemented	85

Modules	86

array	86

Looking for integer not implemented	86

Array deletion not implemented	86

Subscript with step != 1 is not yet implemented	86

builtins	86

Second argument to next() is not implemented	86

deque	87

Deque not implemented	87

json	87

JSON module does not throw exception when object is not serialisable	87

struct	87

Struct pack with too few args, not checked by uPy	87

Struct pack with too many args, not checked by uPy	88

sys	88

Overriding sys.stdin, sys.stdout and sys.stderr not possible	88

License information

The MIT License (MIT)

Copyright (c) 2013-2017 Damien P. George, and others

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the “Software”), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED “AS IS”, WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

ESP32S ESP-32S ESP32 ESP-32 CP2102 Wireless WiFi Bluetooth Development Board Micro USB Dual Core Power Amplifier Filter Module

ESP32 is already integrated antenna and RF balun, power amplifier, low-noise amplifiers, filters, and power management module.
The entire solution takes up the least amount of printed circuit board area.
This board is used with 2.4 GHz dual-mode Wi-Fi and Bluetooth chips by TSMC 40nm low power technology, power and RF properties best, which is safe, reliable, and scalable to a variety of applications.

Features: 
High performance-price ratio
Small volume, easily embeded to other products 
Strong function with support LWIP protocol, Freertos
Supporting three modes: AP, STA, and AP+STA
Supporting Lua program, easily to develop

Development Board: 
https://github.com/espressif/arduino-esp32
https://github.com/nodemcu/nodemcu-firmware/tree/dev-esp32
https://github.com/Nicholas3388/LuaNode

If it cann't get bluetooth working?
Using Node32s as the Board selection. To reference GPIO pins in code use just the number, for example "digitalWrite(13, HIGH)" sets GPIO13 high.
The built-in LED to GPIO2.

Package including: 
1 x ESP32 Development Board

http://docs.micropython.org/en/latest/

https://learn.sparkfun.com/tutorials/micropython-programming-tutorial-getting-started-with-the-esp32-thing/resources-and-going-further

<Note>

In nizhny novgorod the parcel came in 10 days. Works fine!

Attention!! During downloading the firmware you need to press the boot button, otherwise it does not flash. If the board does not connect to the computer, do not rush to throw it out. Try to change the cable to connect. Need a normal cable, not the one that has the charging function

Quick reference for the ESP32

Below is a quick reference for ESP32-based boards.

If it is your first time working with this board it may be useful to get an overview of the microcontroller:

- General information about the ESP32 port

- Getting started with MicroPython on the ESP32

Installing MicroPython

See the corresponding section of tutorial: Getting started with MicroPython on the ESP32.

It also includes a troubleshooting subsection.

General board control

The MicroPython REPL is on UART0 (GPIO1=TX, GPIO3=RX) at baudrate 115200. Tab-completion is useful to find out what methods an object has. Paste mode (ctrl-E) is useful to paste a large slab of Python code into the REPL.

The machine module:

import machine

machine.freq()          # get the current frequency of the CPU

machine.freq(240000000) # set the CPU frequency to 240 MHz

The esp module:

import esp

esp.osdebug(None)       # turn off vendor O/S debugging messages

esp.osdebug(0)          # redirect vendor O/S debugging messages to UART(0)

# low level methods to interact with flash storage

esp.flash_size()

esp.flash_user_start()

esp.flash_erase(sector_no)

esp.flash_write(byte_offset, buffer)

esp.flash_read(byte_offset, buffer)

The esp32 module:

import esp32

esp32.hall_sensor()     # read the internal hall sensor

esp32.raw_temperature() # read the internal temperature of the MCU, in Farenheit

esp32.ULP()             # access to the Ultra-Low-Power Co-processor

Note that the temperature sensor in the ESP32 will typically read higher than ambient due to the IC getting warm while it runs. This effect can be minimized by reading the temperature sensor immediately after waking up from sleep.

Networking

The network module:

import network

wlan = network.WLAN(network.STA_IF) # create station interface

wlan.active(True)       # activate the interface

wlan.scan()             # scan for access points

wlan.isconnected()      # check if the station is connected to an AP

wlan.connect('essid', 'password') # connect to an AP

wlan.config('mac')      # get the interface's MAC address

wlan.ifconfig()         # get the interface's IP/netmask/gw/DNS addresses

ap = network.WLAN(network.AP_IF) # create access-point interface

ap.config(essid='ESP-AP') # set the ESSID of the access point

ap.active(True)         # activate the interface

A useful function for connecting to your local WiFi network is:

def do_connect():

import network

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

if not wlan.isconnected():

print('connecting to network...')

wlan.connect('essid', 'password')

while not wlan.isconnected():

pass

print('network config:', wlan.ifconfig())

Once the network is established the socket module can be used to create and use TCP/UDP sockets as usual, and the urequests module for convenient HTTP requests.

Delay and timing

Use the time module:

import time

time.sleep(1)           # sleep for 1 second

time.sleep_ms(500)      # sleep for 500 milliseconds

time.sleep_us(10)       # sleep for 10 microseconds

start = time.ticks_ms() # get millisecond counter

delta = time.ticks_diff(time.ticks_ms(), start) # compute time difference

Timers

Virtual (RTOS-based) timers are supported. Use the machine.Timer class with timer ID of -1:

from machine import Timer

tim = Timer(-1)

tim.init(period=5000, mode=Timer.ONE_SHOT, callback=lambda t:print(1))

tim.init(period=2000, mode=Timer.PERIODIC, callback=lambda t:print(2))

The period is in milliseconds.

Pins and GPIO

Use the machine.Pin class:

from machine import Pin

p0 = Pin(0, Pin.OUT)    # create output pin on GPIO0

p0.on()                 # set pin to "on" (high) level

p0.off()                # set pin to "off" (low) level

p0.value(1)             # set pin to on/high

p2 = Pin(2, Pin.IN)     # create input pin on GPIO2

print(p2.value())       # get value, 0 or 1

p4 = Pin(4, Pin.IN, Pin.PULL_UP) # enable internal pull-up resistor

p5 = Pin(5, Pin.OUT, value=1) # set pin high on creation

Available Pins are from the following ranges (inclusive): 0-19, 21-23, 25-27, 32-39. These correspond to the actual GPIO pin numbers of ESP32 chip. Note that many end-user boards use their own adhoc pin numbering (marked e.g. D0, D1, ...). For mapping between board logical pins and physical chip pins consult your board documentation.

Notes:

- Pins 1 and 3 are REPL UART TX and RX respectively

- Pins 6, 7, 8, 11, 16, and 17 are used for connecting the embedded flash, and are not recommended for other uses

- Pins 34-39 are input only, and also do not have internal pull-up resistors

- The pull value of some pins can be set to Pin.PULL_HOLD to reduce power consumption during deepsleep.

PWM (pulse width modulation)

PWM can be enabled on all output-enabled pins. The base frequency can range from 1Hz to 40MHz but there is a tradeoff; as the base frequency increases the duty resolution decreases. See LED Control for more details.

Use the machine.PWM class:

from machine import Pin, PWM

pwm0 = PWM(Pin(0))      # create PWM object from a pin

pwm0.freq()             # get current frequency

pwm0.freq(1000)         # set frequency

pwm0.duty()             # get current duty cycle

pwm0.duty(200)          # set duty cycle

pwm0.deinit()           # turn off PWM on the pin

pwm2 = PWM(Pin(2), freq=20000, duty=512) # create and configure in one go

ADC (analog to digital conversion)

On the ESP32 ADC functionality is available on Pins 32-39. Note that, when using the default configuration, input voltages on the ADC pin must be between 0.0v and 1.0v (anything above 1.0v will just read as 4095). Attenuation must be applied in order to increase this usable voltage range.

Use the machine.ADC class:

from machine import ADC

adc = ADC(Pin(32))          # create ADC object on ADC pin

adc.read()                  # read value, 0-4095 across voltage range 0.0v - 1.0v

adc.atten(ADC.ATTN_11DB)    # set 11dB input attenuation (voltage range roughly 0.0v - 3.6v)

adc.width(ADC.WIDTH_9BIT)   # set 9 bit return values (returned range 0-511)

adc.read()                  # read value using the newly configured attenuation and width

ESP32 specific ADC class method reference:

ADC.atten(attenuation)

This method allows for the setting of the amount of attenuation on the input of the ADC. This allows for a wider possible input voltage range, at the cost of accuracy (the same number of bits now represents a wider range). The possible attenuation options are:

- ADC.ATTN_0DB: 0dB attenuation, gives a maximum input voltage of 1.00v - this is the default configuration

- ADC.ATTN_2_5DB: 2.5dB attenuation, gives a maximum input voltage of approximately 1.34v

- ADC.ATTN_6DB: 6dB attenuation, gives a maximum input voltage of approximately 2.00v

- ADC.ATTN_11DB: 11dB attenuation, gives a maximum input voltage of approximately 3.6v

Warning

Despite 11dB attenuation allowing for up to a 3.6v range, note that the absolute maximum voltage rating for the input pins is 3.6v, and so going near this boundary may be damaging to the IC!

ADC.width(width)

This method allows for the setting of the number of bits to be utilised and returned during ADC reads. Possible width options are:

- ADC.WIDTH_9BIT: 9 bit data

- ADC.WIDTH_10BIT: 10 bit data

- ADC.WIDTH_11BIT: 11 bit data

- ADC.WIDTH_12BIT: 12 bit data - this is the default configuration

Software SPI bus

There are two SPI drivers. One is implemented in software (bit-banging) and works on all pins, and is accessed via the machine.SPI class:

from machine import Pin, SPI

# construct an SPI bus on the given pins

# polarity is the idle state of SCK

# phase=0 means sample on the first edge of SCK, phase=1 means the second

spi = SPI(baudrate=100000, polarity=1, phase=0, sck=Pin(0), mosi=Pin(2), miso=Pin(4))

spi.init(baudrate=200000) # set the baudrate

spi.read(10)            # read 10 bytes on MISO

spi.read(10, 0xff)      # read 10 bytes while outputting 0xff on MOSI

buf = bytearray(50)     # create a buffer

spi.readinto(buf)       # read into the given buffer (reads 50 bytes in this case)

spi.readinto(buf, 0xff) # read into the given buffer and output 0xff on MOSI

spi.write(b'12345')     # write 5 bytes on MOSI

buf = bytearray(4)      # create a buffer

spi.write_readinto(b'1234', buf) # write to MOSI and read from MISO into the buffer

spi.write_readinto(buf, buf) # write buf to MOSI and read MISO back into buf

Warning

Currently all of sck, mosi and miso must be specified when initialising Software SPI.

Hardware SPI bus

There are two hardware SPI channels that allow faster transmission rates (up to 80Mhz). These may be used on any IO pins that support the required direction and are otherwise unused (see Pins and GPIO) but if they are not configured to their default pins then they need to pass through an extra layer of GPIO multiplexing, which can impact their reliability at high speeds. Hardware SPI channels are limited to 40MHz when used on pins other than the default ones listed below.

|  | HSPI (id=1) | VSPI (id=2) |
| --- | --- | --- |
| sck | 14 | 18 |
| mosi | 13 | 23 |
| miso | 12 | 19 |

Hardware SPI has the same methods as Software SPI above:

from machine import Pin, SPI

hspi = SPI(1, 10000000, sck=Pin(14), mosi=Pin(13), miso=Pin(12))

vspi = SPI(2, baudrate=80000000, polarity=0, phase=0, bits=8, firstbit=0, sck=Pin(18), mosi=Pin(23), miso=Pin(19))

I2C bus

The I2C driver has both software and hardware implementations, and the two hardware peripherals have identifiers 0 and 1. Any available output-capable pins can be used for SCL and SDA. The driver is accessed via the machine.I2C class:

from machine import Pin, I2C

# construct a software I2C bus

i2c = I2C(scl=Pin(5), sda=Pin(4), freq=100000)

# construct a hardware I2C bus

i2c = I2C(0)

i2c = I2C(1, scl=Pin(5), sda=Pin(4), freq=400000)

i2c.scan()              # scan for slave devices

i2c.readfrom(0x3a, 4)   # read 4 bytes from slave device with address 0x3a

i2c.writeto(0x3a, '12') # write '12' to slave device with address 0x3a

buf = bytearray(10)     # create a buffer with 10 bytes

i2c.writeto(0x3a, buf)  # write the given buffer to the slave

Real time clock (RTC)

See machine.RTC

from machine import RTC

rtc = RTC()

rtc.datetime((2017, 8, 23, 1, 12, 48, 0, 0)) # set a specific date and time

rtc.datetime() # get date and time

Deep-sleep mode

The following code can be used to sleep, wake and check the reset cause:

import machine

# check if the device woke from a deep sleep

if machine.reset_cause() == machine.DEEPSLEEP_RESET:

print('woke from a deep sleep')

# put the device to sleep for 10 seconds

machine.deepsleep(10000)

Notes:

Calling deepsleep() without an argument will put the device to sleep indefinitely

- A software reset does not change the reset cause

- There may be some leakage current flowing through enabled internal pullups. To further reduce power consumption it is possible to disable the internal pullups:

- p1 = Pin(4, Pin.IN, Pin.PULL_HOLD)

After leaving deepsleep it may be necessary to un-hold the pin explicitly (e.g. if it is an output pin) via:

- p1 = Pin(4, Pin.OUT, None)

RMT

The RMT is ESP32-specific and allows generation of accurate digital pulses with 12.5ns resolution. See esp32.RMT for details. Usage is:

import esp32

from machine import Pin

r = esp32.RMT(0, pin=Pin(18), clock_div=8)

r   # RMT(channel=0, pin=18, source_freq=80000000, clock_div=8)

# The channel resolution is 100ns (1/(source_freq/clock_div)).

r.write_pulses((1, 20, 2, 40), start=0) # Send 0 for 100ns, 1 for 2000ns, 0 for 200ns, 1 for 4000ns

OneWire driver

The OneWire driver is implemented in software and works on all pins:

from machine import Pin

import onewire

ow = onewire.OneWire(Pin(12)) # create a OneWire bus on GPIO12

ow.scan()               # return a list of devices on the bus

ow.reset()              # reset the bus

ow.readbyte()           # read a byte

ow.writebyte(0x12)      # write a byte on the bus

ow.write('123')         # write bytes on the bus

ow.select_rom(b'12345678') # select a specific device by its ROM code

There is a specific driver for DS18S20 and DS18B20 devices:

import time, ds18x20

ds = ds18x20.DS18X20(ow)

roms = ds.scan()

ds.convert_temp()

time.sleep_ms(750)

for rom in roms:

print(ds.read_temp(rom))

Be sure to put a 4.7k pull-up resistor on the data line. Note that the convert_temp() method must be called each time you want to sample the temperature.

NeoPixel driver

Use the neopixel module:

from machine import Pin

from neopixel import NeoPixel

pin = Pin(0, Pin.OUT)   # set GPIO0 to output to drive NeoPixels

np = NeoPixel(pin, 8)   # create NeoPixel driver on GPIO0 for 8 pixels

np[0] = (255, 255, 255) # set the first pixel to white

np.write()              # write data to all pixels

r, g, b = np[0]         # get first pixel colour

For low-level driving of a NeoPixel:

import esp

esp.neopixel_write(pin, grb_buf, is800khz)

Warning

By default NeoPixel is configured to control the more popular 800kHz units. It is possible to use alternative timing to control other (typically 400kHz) devices by passing timing=0 when constructing the NeoPixel object.

Capacitive touch

Use the TouchPad class in the machine module:

from machine import TouchPad, Pin

t = TouchPad(Pin(14))

t.read()              # Returns a smaller number when touched

TouchPad.read returns a value relative to the capacitive variation. Small numbers (typically in the tens) are common when a pin is touched, larger numbers (above one thousand) when no touch is present. However the values are relative and can vary depending on the board and surrounding composition so some calibration may be required.

There are ten capacitive touch-enabled pins that can be used on the ESP32: 0, 2, 4, 12, 13 14, 15, 27, 32, 33. Trying to assign to any other pins will result in a ValueError.

Note that TouchPads can be used to wake an ESP32 from sleep:

import machine

from machine import TouchPad, Pin

import esp32

t = TouchPad(Pin(14))

t.config(500)               # configure the threshold at which the pin is considered touched

esp32.wake_on_touch(True)

machine.lightsleep()        # put the MCU to sleep until a touchpad is touched

For more details on touchpads refer to Espressif Touch Sensor.

DHT driver

The DHT driver is implemented in software and works on all pins:

import dht

import machine

d = dht.DHT11(machine.Pin(4))

d.measure()

d.temperature() # eg. 23 (°C)

d.humidity()    # eg. 41 (% RH)

d = dht.DHT22(machine.Pin(4))

d.measure()

d.temperature() # eg. 23.6 (°C)

d.humidity()    # eg. 41.3 (% RH)

WebREPL (web browser interactive prompt)

WebREPL (REPL over WebSockets, accessible via a web browser) is an experimental feature available in ESP32 port. Download web client from https://github.com/micropython/webrepl (hosted version available at http://micropython.org/ webrepl), and configure it by executing:

import webrepl_setup

and following on-screen instructions. After reboot, it will be available for connection. If you disabled automatic start-up on boot, you may run configured daemon on demand using:

import webrepl

webrepl.start()

# or, start with a specific password

webrepl.start(password='mypass')

The WebREPL daemon listens on all active interfaces, which can be STA or AP. This allows you to connect to the ESP32 via a router (the STA interface) or directly when connected to its access point.

In addition to terminal/command prompt access, WebREPL also has provision for file transfer (both upload and download). The web client has buttons for the corresponding functions, or you can use the command-line client webrepl_cli.py from the repository above.

See the MicroPython forum for other community-supported alternatives to transfer files to an ESP32 board.

MicroPython language and implementation

MicroPython aims to implement the Python 3.4 standard (with selected features from later versions) with respect to language syntax, and most of the features of MicroPython are identical to those described by the “Language Reference” documentation at docs.python.org. The MicroPython standard library is described in the corresponding chapter. The MicroPython differences from CPython chapter describes differences between MicroPython and CPython (which mostly concern standard library and types, but also some language-level features).

This chapter describes features and peculiarities of MicroPython implementation and the best practices to use them.

- Glossary

- The MicroPython Interactive Interpreter Mode (aka REPL)

- MicroPython .mpy files

- Writing interrupt handlers

- Maximising MicroPython speed

- MicroPython on microcontrollers

- Distribution packages, package management, and deploying applications

- Inline assembler for Thumb2 architectures

- Working with filesystems

- The pyboard.py tool

Glossary

baremetal

A system without a (full-fledged) operating system, for example an MCU-based system. When running on a baremetal system, MicroPython effectively functions like a small operating system, running user programs and providing a command interpreter (REPL).

buffer protocol

Any Python object that can be automatically converted into bytes, such as bytes, bytearray, memoryview and str objects, which all implement the “buffer protocol”.

board

Typically this refers to a printed circuit board (PCB) containing a microcontroller and supporting components. MicroPython firmware is typically provided per-board, as the firmware contains both MCU-specific functionality but also board-level functionality such as drivers or pin names.

bytecode

A compact representation of a Python program that generated by compiling the Python source code. This is what the VM actually executes. Bytecode is typically generated automatically at runtime and is invisible to the user. Note that while CPython and MicroPython both use bytecode, the format is different. You can also pre-compile source code offline using the cross-compiler.

callee-owned tuple

This is a MicroPython-specific construct where, for efficiency reasons, some built-in functions or methods may re-use the same underlying tuple object to return data. This avoids having to allocate a new tuple for every call, and reduces heap fragmentation. Programs should not hold references to callee-owned tuples and instead only extract data from them (or make a copy).

CircuitPython

A variant of MicroPython developed by Adafruit Industries.

CPython

CPython is the reference implementation of the Python programming language, and the most well-known one. It is, however, one of many implementations (including Jython, IronPython, PyPy, and MicroPython). While MicroPython’s implementation differs substantially from CPython, it aims to maintain as much compatibility as possible.

cross-compiler

Also known as mpy-cross. This tool runs on your PC and converts a .py file containing MicroPython code into a .mpy file containing MicroPython bytecode. This means it loads faster (the board doesn’t have to compile the code), and uses less space on flash (the bytecode is more space efficient).

driver

A MicroPython library that implements support for a particular component, such as a sensor or display.

FFI

Acronym for Foreign Function Interface. A mechanism used by the MicroPython Unix port to access operating system functionality. This is not available on baremetal ports.

filesystem

Most MicroPython ports and boards provide a filesystem stored in flash that is available to user code via the standard Python file APIs such as open(). Some boards also make this internal filesystem accessible to the host via USB mass-storage.

frozen module

A Python module that has been cross compiled and bundled into the firmware image. This reduces RAM requirements as the code is executed directly from flash.

Garbage Collector

A background process that runs in Python (and MicroPython) to reclaim unused memory in the heap.

GPIO

General-purpose input/output. The simplest means to control electrical signals (commonly referred to as “pins”) on a microcontroller. GPIO typically allows pins to be either input or output, and to set or get their digital value (logical “0” or “1”). MicroPython abstracts GPIO access using the machine.Pin and machine.Signal classes.

GPIO port

A group of GPIO pins, usually based on hardware properties of these pins (e.g. controllable by the same register).

heap

A region of RAM where MicroPython stores dynamic data. It is managed automatically by the Garbage Collector. Different MCUs and boards have vastly different amounts of RAM available for the heap, so this will affect how complex your program can be.

interned string

An optimisation used by MicroPython to improve the efficiency of working with strings. An interned string is referenced by its (unique) identity rather than its address and can therefore be quickly compared just by its identifier. It also means that identical strings can be de-duplicated in memory. String interning is almost always invisible to the user.

MCU

Microcontroller. Microcontrollers usually have much less resources than a desktop, laptop, or phone, but are smaller, cheaper and require much less power. MicroPython is designed to be small and optimized enough to run on an average modern microcontroller.

micropython-lib

MicroPython is (usually) distributed as a single executable/binary file with just few builtin modules. There is no extensive standard library comparable with CPython‘s. Instead, there is a related, but separate project micropython-lib which provides implementations for many modules from CPython’s standard library.

Some of the modules are are implemented in pure Python, and are able to be used on all ports. However, the majority of these modules use FFI to access operating system functionality, and as such can only be used on the MicroPython Unix port (with limited support for Windows).

Unlike the CPython stdlib, micropython-lib modules are intended to be installed individually - either using manual copying or using upip.

MicroPython port

MicroPython supports different boards, RTOSes, and OSes, and can be relatively easily adapted to new systems. MicroPython with support for a particular system is called a “port” to that system. Different ports may have widely different functionality. This documentation is intended to be a reference of the generic APIs available across different ports (“MicroPython core”). Note that some ports may still omit some APIs described here (e.g. due to resource constraints). Any such differences, and port-specific extensions beyond the MicroPython core functionality, would be described in the separate port-specific documentation.

MicroPython Unix port

The unix port is one of the major MicroPython ports. It is intended to run on POSIX-compatible operating systems, like Linux, MacOS, FreeBSD, Solaris, etc. It also serves as the basis of Windows port. The Unix port is very useful for quick development and testing of the MicroPython language and machine-independent features. It can also function in a similar way to CPython‘s python executable.

.mpy file

The output of the cross-compiler. A compiled form of a .py file that contains MicroPython bytecode instead of Python source code.

native

Usually refers to “native code”, i.e. machine code for the target microcontroller (such as ARM Thumb, Xtensa, x86/x64). The @native decorator can be applied to a MicroPython function to generate native code instead of bytecode for that function, which will likely be faster but use more RAM.

port

Usually short for MicroPython port, but could also refer to GPIO port.

.py file

A file containing Python source code.

REPL

An acronym for “Read, Eval, Print, Loop”. This is the interactive Python prompt, useful for debugging or testing short snippets of code. Most MicroPython boards make a REPL available over a UART, and this is typically accessible on a host PC via USB.

stream

Also known as a “file-like object”. An Python object which provides sequential read-write access to the underlying data. A stream object implements a corresponding interface, which consists of methods like read(),  write(), readinto(),  seek(),

flush(), close(), etc. A stream is an important concept in MicroPython; many I/O objects implement the stream interface, and thus can be used consistently and interchangeably in different contexts. For more information on streams in MicroPython, see the uio module.

UART

Acronym for “Universal Asynchronous Receiver/Transmitter”. This is a peripheral that sends data over a pair of pins (TX & RX). Many boards include a way to make at least one of the UARTs available to a host PC as a serial port over USB.

upip

(Literally, “micro pip”). A package manager for MicroPython, inspired by CPython‘s pip, but much smaller and with reduced functionality. upip runs both on the Unix port and on baremetal ports which offer filesystem and networking support.

The MicroPython Interactive Interpreter Mode (aka REPL)

This section covers some characteristics of the MicroPython Interactive Interpreter Mode. A commonly used term for this is REPL (read-eval-print-loop) which will be used to refer to this interactive prompt.

Auto-indent

When typing python statements which end in a colon (for example if, for, while) then the prompt will change to three dots (...) and the cursor will be indented by 4 spaces. When you press return, the next line will continue at the same level of indentation for regular statements or an additional level of indentation where appropriate. If you press the backspace key then it will undo one level of indentation.

If your cursor is all the way back at the beginning, pressing RETURN will then execute the code that you’ve entered. The following shows what you’d see after entering a for statement (the underscore shows where the cursor winds up):

>>> for i in range(30):

...     _

If you then enter an if statement, an additional level of indentation will be provided:

>>> for i in range(30):

...     if i > 3:

...         _

Now enter break followed by RETURN and press BACKSPACE:

>>> for i in range(30):

...     if i > 3:

...         break

...     _

Finally type print(i), press RETURN, press BACKSPACE and press RETURN again:

>>> for i in range(30):

...     if i > 3:

...         break

...     print(i)

...

0

1

2

3

>>>

Auto-indent won’t be applied if the previous two lines were all spaces. This means that you can finish entering a compound statement by pressing RETURN twice, and then a third press will finish and execute.

Auto-completion

While typing a command at the REPL, if the line typed so far corresponds to the beginning of the name of something, then pressing TAB will show possible things that could be entered. For example, first import the machine module by entering import machine and pressing RETURN. Then type m and press TAB and it should expand to machine. Enter a dot . and press TAB again. You should see something like:

>>> machine.

__name__        info            unique_id       reset

bootloader      freq            rng             idle

sleep           deepsleep       disable_irq     enable_irq

Pin

The word will be expanded as much as possible until multiple possibilities exist. For example, type machine.Pin.AF3 and press TAB and it will expand to machine.Pin.AF3_TIM. Pressing TAB a second time will show the possible expansions:

>>> machine.Pin.AF3_TIM

AF3_TIM10       AF3_TIM11       AF3_TIM8        AF3_TIM9

>>> machine.Pin.AF3_TIM

Interrupting a running program

You can interrupt a running program by pressing Ctrl-C. This will raise a KeyboardInterrupt which will bring you back to the REPL, providing your program doesn’t intercept the KeyboardInterrupt exception.

For example:

>>> for i in range(1000000):

...     print(i)

...

0

1

2

3

...

6466

6467

6468

Traceback (most recent call last):

File "<stdin>", line 2, in <module>

KeyboardInterrupt:

>>>

Paste mode

If you want to paste some code into your terminal window, the auto-indent feature will mess things up. For example, if you had the following python code:

def foo():

print('This is a test to show paste mode')

print('Here is a second line')

foo()

and you try to paste this into the normal REPL, then you will see something like this:

>>> def foo():

...         print('This is a test to show paste mode')

...             print('Here is a second line')

...             foo()

...

File "<stdin>", line 3

IndentationError: unexpected indent

If you press Ctrl-E, then you will enter paste mode, which essentially turns off the auto-indent feature, and changes the prompt from >>> to ===. For example:

>>>

paste mode; Ctrl-C to cancel, Ctrl-D to finish

=== def foo():

===     print('This is a test to show paste mode')

===     print('Here is a second line')

=== foo()

===

This is a test to show paste mode

Here is a second line

>>>

Paste Mode allows blank lines to be pasted. The pasted text is compiled as if it were a file. Pressing Ctrl-D exits paste mode and initiates the compilation.

Soft reset

A soft reset will reset the python interpreter, but tries not to reset the method by which you’re connected to the MicroPython board (USB-serial, or Wifi).

You can perform a soft reset from the REPL by pressing Ctrl-D, or from your python code by executing:

machine.soft_reset()

For example, if you reset your MicroPython board, and you execute a dir() command, you’d see something like this:

>>> dir()

['__name__', 'pyb']

Now create some variables and repeat the dir() command:

>>> i = 1

>>> j = 23

>>> x = 'abc'

>>> dir()

['j', 'x', '__name__', 'pyb', 'i']

>>>

Now if you enter Ctrl-D, and repeat the dir() command, you’ll see that your variables no longer exist:

MPY: sync filesystems

MPY: soft reboot

MicroPython v1.5-51-g6f70283-dirty on 2015-10-30; PYBv1.0 with STM32F405RG

Type "help()" for more information.

>>> dir()

['__name__', 'pyb']

>>>

The special variable _ (underscore)

When you use the REPL, you may perform computations and see the results. MicroPython stores the results of the previous statement in the variable _ (underscore). So you can use the underscore to save the result in a variable. For example:

>>> 1 + 2 + 3 + 4 + 5

15

>>> x = _

>>> x

15

>>>

Raw mode

Raw mode is not something that a person would normally use. It is intended for programmatic use. It essentially behaves like paste mode with echo turned off.

Raw mode is entered using Ctrl-A. You then send your python code, followed by a Ctrl-D. The Ctrl-D will be acknowledged by ‘OK’ and then the python code will be compiled and executed. Any output (or errors) will be sent back. Entering Ctrl-B will leave raw mode and return the the regular (aka friendly) REPL.

The tools/pyboard.py program uses the raw REPL to execute python files on the MicroPython board.

MicroPython .mpy files

MicroPython defines the concept of an .mpy file which is a binary container file format that holds precompiled code, and which can be imported like a normal .py module. The file foo.mpy can be imported via import foo, as long as foo.mpy can be found in the usual way by the import machinery. Usually, each directory listed in sys.path is searched in order. When searching a particular directory foo.py is looked for first and if that is not found then foo.mpy is looked for, then the search continues in the next directory if neither is found. As such, foo.py will take precedence over foo.mpy.

These .mpy files can contain bytecode which is usually generated from Python source files (.py files) via the mpy-cross program. For some architectures an .mpy file can also contain native machine code, which can be generated in a variety of ways, most notably from C source code.

Versioning and compatibility of .mpy files

A given .mpy file may or may not be compatible with a given MicroPython system. Compatibility is based on the following:

- Version of the .mpy file: the version of the file must match the version supported by the system loading it.

- Bytecode features used in the .mpy file: there are two bytecode features which must match between the file and the system: unicode support and inline caching of map lookups in the bytecode.

- Small integer bits: the .mpy file will require a minimum number of bits in a small integer and the system loading it must support at least this many bits.

- Qstr compression window size: the .mpy file will require a minimum window size for qstr decompression and the system loading it must have a window greater or equal to this size.

- Native architecture: if the .mpy file contains native machine code then it will specify the architecture of that machine code and the system loading it must support execution of that architecture’s code.

If a MicroPython system supports importing .mpy files then the sys.implementation.mpy field will exist and return an integer which encodes the version (lower 8 bits), features and native architecture.

Trying to import an .mpy file that fails one of the first four tests will raise ValueError('incompatible .mpy file'). Trying to import an .mpy file that fails the native architecture test (if it contains native machine code) will raise ValueError('incompatible .mpy arch').

If importing an .mpy file fails then try the following:

Determine the .mpy version and flags supported by your MicroPython system by executing:

import sys

sys_mpy = sys.implementation.mpy

arch = [None, 'x86', 'x64',

'armv6', 'armv6m', 'armv7m', 'armv7em', 'armv7emsp', 'armv7emdp',

'xtensa', 'xtensawin'][sys_mpy >> 10]

print('mpy version:', sys_mpy & 0xff)

print('mpy flags:', end='')

if arch:

print(' -march=' + arch, end='')

if sys_mpy & 0x100:

print(' -mcache-lookup-bc', end='')

if not sys_mpy & 0x200:

print(' -mno-unicode', end='')

print()

- Check the validity of the .mpy file by inspecting the first two bytes of the file. The first byte should be an uppercase ‘M’ and the second byte will be the version number, which should match the system version from above. If it doesn’t match then rebuild the .mpy file.

- Check if the system .mpy version matches the version emitted by mpy-cross that was used to build the .mpy file, found by mpy-cross --version. If it doesn’t match then recompile mpy-cross from the Git repository checked out at the tag (or hash) reported by mpy-cross --version.

- Make sure you are using the correct mpy-cross flags, found by the code above, or by inspecting the MPY_CROSS_FLAGS Makefile variable for the port that you are using.

The following table shows the correspondence between MicroPython release and .mpy version.

| MicroPython release | .mpy version |
| --- | --- |
| v1.12 and up | 5 |
| v1.11 | 4 |
| v1.9.3 - v1.10 | 3 |
| v1.9 - v1.9.2 | 2 |
| v1.5.1 - v1.8.7 | 0 |

For completeness, the next table shows the Git commit of the main MicroPython repository at which the .mpy version was changed.

| .mpy version change | Git commit |
| --- | --- |
| 4 to 5 | 5716c5cf65e9b2cb46c2906f40302401bdd27517 |
| 3 to 4 | 9a5f92ea72754c01cc03e5efcdfe94021120531e |
| 2 to 3 | ff93fd4f50321c6190e1659b19e64fef3045a484 |
| 1 to 2 | dd11af209d226b7d18d5148b239662e30ed60bad |
| 0 to 1 | 6a11048af1d01c78bdacddadd1b72dc7ba7c6478 |
| initial version 0 | d8c834c95d506db979ec871417de90b7951edc30 |

Binary encoding of .mpy files

MicroPython .mpy files are a binary container format with code objects stored internally in a nested hierarchy. To keep files small while still providing a large range of possible values it uses the concept of a variably-encoded-unsigned-integer (vuint) in many places. Similar to utf-8 encoding, this encoding stores 7 bits per byte with the 8th bit (MSB) set if one or more bytes follow. The bits of the unsigned integer are stored in the vuint in LSB form.

The top-level of an .mpy file consists of two parts:

- The header.

- The raw-code for the outer scope of the module. This outer scope is executed when the .mpy file is imported.

- The header

The .mpy header is:

| size | field |
| --- | --- |
| byte | value 0x4d (ASCII ‘M’) |
| byte | .mpy version number |
| byte | feature flags |
| byte | number of bits in a small int |
| vuint | size of qstr window |

Raw code elements

A raw-code element contains code, either bytecode or native machine code. Its contents are:

| size | field |
| --- | --- |
| vuint | type and size |
| ... | code (bytecode or machine code) |
| vuint | number of constant objects |
| vuint | number of sub-raw-code elements |
| ... | constant objects |
| ... | sub-raw-code elements |

The first vuint in a raw-code element encodes the type of code stored in this element (the two least-significant bits), and the decompressed length of the code (the amount of RAM to allocate for it).

Following the vuint comes the code itself. In the case of bytecode it also contains compressed qstr values.

Following the code comes a vuint counting the number of constant objects, and another vuint counting the number of sub-raw-code elements.The constant objects are then stored next. Finally any sub-raw-code elements are stored, recursively.

Writing interrupt handlers

On suitable hardware MicroPython offers the ability to write interrupt handlers in Python. Interrupt handlers - also known as interrupt service routines (ISR’s) - are defined as callback functions. These are executed in response to an event such as a timer trigger or a voltage change on a pin. Such events can occur at any point in the execution of the program code. This carries significant consequences, some specific to the MicroPython language. Others are common to all systems capable of responding to real time events. This document covers the language specific issues first, followed by a brief introduction to real time programming for those new to it.

This introduction uses vague terms like “slow” or “as fast as possible”. This is deliberate, as speeds are application dependent. Acceptable durations for an ISR are dependent on the rate at which interrupts occur, the nature of the main program, and the presence of other concurrent events.

Tips and recommended practices

This summarises the points detailed below and lists the principal recommendations for interrupt handler code.

- Keep the code as short and simple as possible.

- Avoid memory allocation: no appending to lists or insertion into dictionaries, no floating point.

- Consider using micropython.schedule to work around the above constraint.

- Where an ISR returns multiple bytes use a pre-allocated bytearray. If multiple integers are to be shared between an ISR and the main program consider an array (array.array).

- Where data is shared between the main program and an ISR, consider disabling interrupts prior to accessing the data in the main program and re-enabling them immediately afterwards (see Critical Sections).

- Allocate an emergency exception buffer (see below).

MicroPython issues

The emergency exception buffer

If an error occurs in an ISR, MicroPython is unable to produce an error report unless a special buffer is created for the purpose. Debugging is simplified if the following code is included in any program using interrupts.

import micropython

micropython.alloc_emergency_exception_buf(100)

Simplicity

For a variety of reasons it is important to keep ISR code as short and simple as possible. It should do only what has to be done immediately after the event which caused it: operations which can be deferred should be delegated to the main program loop. Typically an ISR will deal with the hardware device which caused the interrupt, making it ready for the next interrupt to occur. It will communicate with the main loop by updating shared data to indicate that the interrupt has occurred, and it will return. An ISR should return control to the main loop as quickly as possible. This is not a specific MicroPython issue so is covered in more detail below.

Communication between an ISR and the main program

Normally an ISR needs to communicate with the main program. The simplest means of doing this is via one or more shared data objects, either declared as global or shared via a class (see below). There are various restrictions and hazards around doing this, which are covered in more detail below. Integers, bytes and bytearray objects are commonly used for this purpose along with arrays (from the array module) which can store various data types.

The use of object methods as callbacks

MicroPython supports this powerful technique which enables an ISR to share instance variables with the underlying code. It also enables a class implementing a device driver to support multiple device instances. The following example causes two LED’s to flash at different rates.

import pyb, micropython

micropython.alloc_emergency_exception_buf(100)

class Foo(object):

def __init__(self, timer, led):

self.led = led

timer.callback(self.cb)

def cb(self, tim):

self.led.toggle()

red = Foo(pyb.Timer(4, freq=1), pyb.LED(1))

green = Foo(pyb.Timer(2, freq=0.8), pyb.LED(2))

In this example the red instance associates timer 4 with LED 1: when a timer 4 interrupt occurs red.cb() is called causing LED 1 to change state. The green instance operates similarly: a timer 2 interrupt results in the execution of green.cb() and toggles LED 2. The use of instance methods confers two benefits. Firstly a single class enables code to be shared between multiple hardware instances. Secondly, as a bound method the callback function’s first argument is self. This enables the callback to access instance data and to save state between successive calls. For example,

if the class above had a variable self.count set to zero in the constructor, cb() could increment the counter. The red and green instances would then maintain independent counts of the number of times each LED had changed state.

Creation of Python objects

ISR’s cannot create instances of Python objects. This is because MicroPython needs to allocate memory for the object from a store of free memory block called the heap. This is not permitted in an interrupt handler because heap allocation is not re-entrant. In other words the interrupt might occur when the main program is part way through performing an allocation - to maintain the integrity of the heap the interpreter disallows memory allocations in ISR code.

A consequence of this is that ISR’s can’t use floating point arithmetic; this is because floats are Python objects. Similarly an ISR can’t append an item to a list. In practice it can be hard to determine exactly which code constructs will attempt to perform memory allocation and provoke an error message: another reason for keeping ISR code short and simple.

One way to avoid this issue is for the ISR to use pre-allocated buffers. For example a class constructor creates a bytearray instance and a boolean flag. The ISR method assigns data to locations in the buffer and sets the flag. The memory allocation occurs in the main program code when the object is instantiated rather than in the ISR.

The MicroPython library I/O methods usually provide an option to use a pre-allocated buffer.

For example pyb.i2c.recv() can accept a mutable buffer as its first argument: this enables its use in an ISR.

A means of creating an object without employing a class or globals is as follows:

def set_volume(t, buf=bytearray(3)):

buf[0] = 0xa5

buf[1] = t >> 4

buf[2] = 0x5a

return buf

The compiler instantiates the default buf argument when the function is loaded for the first time (usually when the module it’s in is imported).

An instance of object creation occurs when a reference to a bound method is created. This means that an ISR cannot pass a bound method to a function. One solution is to create a reference to the bound method in the class constructor and to pass that reference in the ISR. For example:

class Foo():

def __init__(self):

self.bar_ref = self.bar  # Allocation occurs here

self.x = 0.1

tim = pyb.Timer(4)

tim.init(freq=2)

tim.callback(self.cb)

def bar(self, _):

self.x *= 1.2

print(self.x)

def cb(self, t):

# Passing self.bar would cause allocation.

micropython.schedule(self.bar_ref, 0)

Other techniques are to define and instantiate the method in the constructor or to pass Foo.bar() with the argument self.

Use of Python objects

A further restriction on objects arises because of the way Python works. When an import statement is executed the Python code is compiled to bytecode, with one line of code typically mapping to multiple bytecodes. When the code runs the interpreter reads each bytecode and executes it as a series of machine code instructions. Given that an interrupt can occur at any time between machine code instructions, the original line of Python code may be only partially executed. Consequently a Python object such as a set, list or dictionary modified in the main loop may lack internal consistency at the moment the interrupt occurs.

A typical outcome is as follows. On rare occasions the ISR will run at the precise moment in time when the object is partially updated. When the ISR tries to read the object, a crash results. Because such problems typically occur on rare, random occasions they can be hard to diagnose. There are ways to circumvent this issue, described in Critical Sections below.

It is important to be clear about what constitutes the modification of an object. An alteration to a built-in type such as a dictionary is problematic. Altering the contents of an array or bytearray is not. This is because bytes or words are written as a single machine code instruction which is not interruptible: in the parlance of real time programming the write is atomic. A user defined object might instantiate an integer, array or bytearray. It is valid for both the main loop and the ISR to alter the contents of these.

MicroPython supports integers of arbitrary precision. Values between 2**30 -1 and -2**30 will be stored in a single machine word. Larger values are stored as Python objects. Consequently changes to long integers cannot be considered atomic. The use of long integers in ISR’s is unsafe because memory allocation may be attempted as the variable’s value changes.

Overcoming the float limitation

In general it is best to avoid using floats in ISR code: hardware devices normally handle integers and conversion to floats is normally done in the main loop. However there are a few DSP algorithms which require floating point. On platforms with hardware floating point (such as the Pyboard) the inline ARM Thumb assembler can be used to work round this limitation. This is because the processor stores float values in a machine word; values can be shared between the ISR and main program code via an array of floats.

Using micropython.schedule

This function enables an ISR to schedule a callback for execution “very soon”. The callback is queued for execution which will take place at a time when the heap is not locked. Hence it can create Python objects and use floats. The callback is also guaranteed to run at a time when the main program has completed any update of Python objects, so the callback will not encounter partially updated objects.

Typical usage is to handle sensor hardware. The ISR acquires data from the hardware and enables it to issue a further interrupt. It then schedules a callback to process the data.

Scheduled callbacks should comply with the principles of interrupt handler design outlined below. This is to avoid problems resulting from I/O activity and the modification of shared data which can arise in any code which pre-empts the main program loop.

Execution time needs to be considered in relation to the frequency with which interrupts can occur. If an interrupt occurs while the previous callback is executing, a further instance of the callback will be queued for execution; this will run after the current instance has completed. A sustained high interrupt repetition rate therefore carries a risk of unconstrained queue growth and eventual failure with a RuntimeError.

If the callback to be passed to schedule() is a bound method, consider the note in “Creation of Python objects”.

Exceptions

If an ISR raises an exception it will not propagate to the main loop. The interrupt will be disabled unless the exception is handled by the ISR code.

General issues

This is merely a brief introduction to the subject of real time programming. Beginners should note that design errors in real time programs can lead to faults which are particularly hard to diagnose. This is because they can occur rarely and at intervals which are essentially random. It is crucial to get the initial design right and to anticipate issues before they arise. Both interrupt handlers and the main program need to be designed with an appreciation of the following issues.

Interrupt handler design

As mentioned above, ISR’s should be designed to be as simple as possible. They should always return in a short, predictable period of time. This is important because when the ISR is running, the main loop is not: inevitably the main loop experiences pauses in its execution at random points in the code. Such pauses can be a source of hard to diagnose bugs particularly if their duration is long or variable. In order to understand the implications of ISR run time, a basic grasp of interrupt priorities is required.

Interrupts are organised according to a priority scheme. ISR code may itself be interrupted by a higher priority interrupt. This has implications if the two interrupts share data (see Critical Sections below). If such an interrupt occurs it interposes a delay into the ISR code. If a lower priority interrupt occurs while the ISR is running, it will be delayed until the ISR is complete: if the delay is too long, the lower priority interrupt may fail. A further issue with slow ISR’s is the case where a second interrupt of the same type occurs during its execution. The second interrupt will be handled on termination of the first. However if the rate of incoming interrupts consistently exceeds the capacity of the ISR to service them the outcome will not be a happy one.

Consequently looping constructs should be avoided or minimised. I/O to devices other than to the interrupting device should normally be avoided: I/O such as disk access, print statements and UART access is relatively slow, and its duration may vary. A further issue here is that filesystem functions are not reentrant: using filesystem I/O in an ISR and the main program would be hazardous. Crucially ISR code should not wait on an event. I/O is acceptable if the code can be guaranteed to return in a predictable period, for example toggling a pin or LED. Accessing the interrupting device via I2C or SPI may be necessary but the time taken for such accesses should be calculated or measured and its impact on the application assessed.

There is usually a need to share data between the ISR and the main loop. This may be done either through global variables or via class or instance variables. Variables are typically integer or boolean types, or integer or byte arrays (a pre-allocated integer array offers faster access than a list). Where multiple values are modified by the ISR it is necessary to consider the case where the interrupt occurs at a time when the main program has accessed some, but not all, of the values. This can lead to inconsistencies.

Consider the following design. An ISR stores incoming data in a bytearray, then adds the number of bytes received to an integer representing total bytes ready for processing. The main program reads the number of bytes, processes the bytes, then clears down the number of bytes ready. This will work until an interrupt occurs just after the main program has read the number of bytes. The ISR puts the added data into the buffer and updates the number received, but the main program has already read the number, so processes the data originally received. The newly arrived bytes are lost.

There are various ways of avoiding this hazard, the simplest being to use a circular buffer. If it is not possible to use a structure with inherent thread safety other ways are described below.

Reentrancy

A potential hazard may occur if a function or method is shared between the main program and one or more ISR’s or between multiple ISR’s. The issue here is that the function may itself be interrupted and a further instance of that function run. If this is to occur, the function must be designed to be reentrant. How this is done is an advanced topic beyond the scope of this tutorial.

Critical sections

An example of a critical section of code is one which accesses more than one variable which can be affected by an ISR. If the interrupt happens to occur between accesses to the individual variables, their values will be inconsistent. This is an instance of a hazard known as a race condition: the ISR and the main program loop race to alter the variables. To avoid inconsistency a means must be employed to ensure that the ISR does not alter the values for the duration of the critical section. One way to achieve this is to issue pyb.disable_irq() before the start of the section, and pyb.enable_irq() at the end. Here is an example of this approach:

import pyb, micropython, array

micropython.alloc_emergency_exception_buf(100)

class BoundsException(Exception):

pass

ARRAYSIZE = const(20)

index = 0

data = array.array('i', 0 for x in range(ARRAYSIZE))

def callback1(t):

global data, index

for x in range(5):

data[index] = pyb.rng() # simulate input

index += 1

if index >= ARRAYSIZE:

raise BoundsException('Array bounds exceeded')

tim4 = pyb.Timer(4, freq=100, callback=callback1)

for loop in range(1000):

if index > 0:

irq_state = pyb.disable_irq() # Start of critical section

for x in range(index):

print(data[x])

index = 0

pyb.enable_irq(irq_state) # End of critical section

print('loop {}'.format(loop))

pyb.delay(1)

tim4.callback(None)

A critical section can comprise a single line of code and a single variable. Consider the following code fragment.

count = 0

def cb(): # An interrupt callback

count +=1

def main():

# Code to set up the interrupt callback omitted

while True:

count += 1

This example illustrates a subtle source of bugs. The line count += 1 in the main loop carries a specific race condition hazard known as a read-modify-write. This is a classic cause of bugs in real time systems. In the main loop MicroPython reads the value of t.counter, adds 1 to it, and writes it back. On rare occasions the interrupt occurs after the read and before the write. The interrupt modifies t.counter but its change is overwritten by the main loop when the ISR returns. In a real system this could lead to rare, unpredictable failures.

As mentioned above, care should be taken if an instance of a Python built in type is modified in the main code and that instance is accessed in an ISR. The code performing the modification should be regarded as a critical section to ensure that the instance is in a valid state when the ISR runs.

Particular care needs to be taken if a dataset is shared between different ISR’s. The hazard here is that the higher priority interrupt may occur when the lower priority one has partially updated the shared data. Dealing with this situation is an advanced topic beyond the scope of this introduction other than to note that mutex objects described below can sometimes be used.

Disabling interrupts for the duration of a critical section is the usual and simplest way to proceed, but it disables all interrupts rather than merely the one with the potential to cause problems. It is generally undesirable to disable an interrupt for long. In the case of timer interrupts it introduces variability to the time when a callback occurs. In the case of device interrupts, it can lead to the device being serviced too late with possible loss of data or overrun errors in the device hardware. Like ISR’s, a critical section in the main code should have a short, predictable duration.

An approach to dealing with critical sections which radically reduces the time for which interrupts are disabled is to use an object termed a mutex (name derived from the notion of mutual exclusion). The main program locks the mutex before running the critical section and unlocks it at the end. The ISR tests whether the mutex is locked. If it is, it avoids the critical section and returns. The design challenge is defining what the ISR should do in the event that access to the critical variables is denied. A simple example of a mutex may be found here. Note that the mutex code does disable interrupts, but only for the duration of eight machine instructions: the benefit of this approach is that other interrupts are virtually unaffected.

Interrupts and the REPL

Interrupt handlers, such as those associated with timers, can continue to run after a program terminates. This may produce unexpected results where you might have expected the object raising the callback to have gone out of scope. For example on the Pyboard:

def bar():

foo = pyb.Timer(2, freq=4, callback=lambda t: print('.', end=''))

bar()

This continues to run until the timer is explicitly disabled or the board is reset with ctrl D.

Maximizing MicroPython speed

This tutorial describes ways of improving the performance of MicroPython code. Optimisations involving other languages are covered elsewhere, namely the use of modules written in C and the MicroPython inline assembler.

Designing for speed

Performance issues should be considered at the outset. This involves taking a view on the sections of code which are most performance critical and devoting particular attention to their design. The process of optimisation begins when the code has been tested: if the design is correct at the outset optimisation will be straightforward and may actually be unnecessary.

Algorithms

The most important aspect of designing any routine for performance is ensuring that the best algorithm is employed. This is a topic for textbooks rather than for a MicroPython guide but spectacular performance gains can sometimes be achieved by adopting algorithms known for their efficiency.

RAM allocation

To design efficient MicroPython code it is necessary to have an understanding of the way the interpreter allocates RAM. When an object is created or grows in size (for example where an item is appended to a list) the necessary RAM is allocated from a block known as the heap. This takes a significant amount of time; further it will on occasion trigger a process known as garbage collection which can take several milliseconds.

Consequently the performance of a function or method can be improved if an object is created once only and not permitted to grow in size. This implies that the object persists for the duration of its use: typically it will be instantiated in a class constructor and used in various methods.

This is covered in further detail Controlling garbage collection below.

Buffers

An example of the above is the common case where a buffer is required, such as one used for communication with a device. A typical driver will create the buffer in the constructor and use it in its I/O methods which will be called repeatedly.

The MicroPython libraries typically provide support for pre-allocated buffers. For example, objects which support stream interface (e.g., file or UART) provide read() method which allocates new buffer for read data, but also a readinto() method to read data into an existing buffer.

Floating point

Some MicroPython ports allocate floating point numbers on heap. Some other ports may lack dedicated floating-point coprocessor, and perform arithmetic operations on them in “software” at considerably lower speed than on integers. Where performance is important, use integer operations and restrict the use of floating point to sections of the code where performance is not paramount. For example, capture ADC readings as integers values to an array in one quick go, and only then convert them to floating-point numbers for signal processing.

Arrays

Consider the use of the various types of array classes as an alternative to lists. The array module supports various element types with 8-bit elements supported by Python’s built in bytes and bytearray classes. These data structures all store elements in contiguous memory locations. Once again to avoid memory allocation in critical code these should be pre-allocated and passed as arguments or as bound objects.

When passing slices of objects such as bytearray instances, Python creates a copy which involves allocation of the size proportional to the size of slice. This can be alleviated using a memoryview object. memoryview itself is allocated on heap, but is a small, fixed-size object, regardless of the size of slice it points too.

ba = bytearray(10000)  # big array

func(ba[30:2000])      # a copy is passed, ~2K new allocation

mv = memoryview(ba)    # small object is allocated

func(mv[30:2000])      # a pointer to memory is passed

A memoryview can only be applied to objects supporting the buffer protocol - this includes arrays but not lists. Small caveat is that while memoryview object is live, it also keeps alive the original buffer object. So, a memoryview isn’t a universal panacea. For instance, in the example above, if you are done with 10K buffer and just need those bytes 30:2000 from it, it may be better to make a slice, and let the 10K buffer go (be ready for garbage collection), instead of making a long-living memoryview and keeping 10K blocked for GC.

Nonetheless, memoryview is indispensable for advanced preallocated buffer management. readinto() method discussed above puts data at the beginning of buffer and fills in entire buffer. What if you need to put data in the middle of existing buffer? Just create a memoryview into the needed section of buffer and pass it to readinto().

Identifying the slowest section of code

This is a process known as profiling and is covered in textbooks and (for standard Python) supported by various software tools. For the type of smaller embedded application likely to be running on MicroPython platforms the slowest function or method can usually be established by judicious use of the timing ticks group of functions documented in utime. Code execution time can be measured in ms, us, or CPU cycles.

The following enables any function or method to be timed by adding an @timed_function decorator:

def timed_function(f, *args, **kwargs):

myname = str(f).split(' ')[1]

def new_func(*args, **kwargs):

t = utime.ticks_us()

result = f(*args, **kwargs)

delta = utime.ticks_diff(utime.ticks_us(), t)

print('Function {} Time = {:6.3f}ms'.format(myname, delta/1000))

return result

return new_func

MicroPython code improvements

The const() declaration

MicroPython provides a const() declaration. This works in a similar way to #define in C in that when the code is compiled to bytecode the compiler substitutes the numeric value for the identifier. This avoids a dictionary lookup at runtime. The argument to const() may be anything which, at compile time, evaluates to an integer e.g. 0x100 or 1 << 8.

Caching object references

Where a function or method repeatedly accesses objects performance is improved by caching the object in a local variable:

class foo(object):

def __init__(self):

self.ba = bytearray(100)

def bar(self, obj_display):

ba_ref = self.ba

fb = obj_display.framebuffer

# iterative code using these two objects

This avoids the need repeatedly to look up self.ba and obj_display.framebuffer in the body of the method bar().

Controlling garbage collection

When memory allocation is required, MicroPython attempts to locate an adequately sized block on the heap. This may fail, usually because the heap is cluttered with objects which are no longer referenced by code. If a failure occurs, the process known as garbage collection reclaims the memory used by these redundant objects and the allocation is then tried again - a process which can take several milliseconds.

There may be benefits in pre-empting this by periodically issuing gc.collect(). Firstly doing a collection before it is actually required is quicker - typically on the order of 1ms if done frequently. Secondly you can determine the point in code where this time is used rather than have a longer delay occur at random points, possibly in a speed critical section. Finally performing collections regularly can reduce fragmentation in the heap. Severe fragmentation can lead to non-recoverable allocation failures.

The Native code emitter

This causes the MicroPython compiler to emit native CPU opcodes rather than bytecode. It covers the bulk of the MicroPython functionality, so most functions will require no adaptation (but see below). It is invoked by means of a function decorator:

@micropython.native

def foo(self, arg):

buf = self.linebuf # Cached object

# code

There are certain limitations in the current implementation of the native code emitter.

- Context managers are not supported (the with statement).

- Generators are not supported.

- If raise is used an argument must be supplied.

The trade-off for the improved performance (roughly twice as fast as bytecode) is an increase in compiled code size.

The Viper code emitter

The optimisations discussed above involve standards-compliant Python code. The Viper code emitter is not fully compliant. It supports special Viper native data types in pursuit of performance. Integer processing is non-compliant because it uses machine words: arithmetic on 32 bit hardware is performed modulo 2**32.

Like the Native emitter Viper produces machine instructions but further optimisations are performed, substantially increasing performance especially for integer arithmetic and bit manipulations. It is invoked using a decorator:

@micropython.viper

def foo(self, arg: int) -> int:

# code

As the above fragment illustrates it is beneficial to use Python type hints to assist the Viper optimiser. Type hints provide information on the data types of arguments and of the return value; these are a standard Python language feature formally defined here PEP0484. Viper supports its own set of types namely int, uint (unsigned integer), ptr, ptr8, ptr16 and

ptr32. The ptrX types are discussed below. Currently the uint type serves a single purpose: as a type hint for a function return value. If such a function returns 0xffffffff Python will interpret the result as 2**32 -1 rather than as -1.

In addition to the restrictions imposed by the native emitter the following constraints apply:

- Functions may have up to four arguments.

- Default argument values are not permitted.

- Floating point may be used but is not optimised.

Viper provides pointer types to assist the optimiser. These comprise

- ptr Pointer to an object.

- ptr8 Points to a byte.

- ptr16 Points to a 16 bit half-word.

- ptr32 Points to a 32 bit machine word.

The concept of a pointer may be unfamiliar to Python programmers. It has similarities to a Python memoryview object in that it provides direct access to data stored in memory. Items are accessed using subscript notation, but slices are not supported: a pointer can return a single item only. Its purpose is to provide fast random access to data stored in contiguous memory locations - such as data stored in objects which support the buffer protocol, and memory-mapped peripheral registers in a microcontroller. It should be noted that programming using pointers is hazardous: bounds checking is not performed and the compiler does nothing to prevent buffer overrun errors.

Typical usage is to cache variables:

@micropython.viper

def foo(self, arg: int) -> int:

buf = ptr8(self.linebuf) # self.linebuf is a bytearray or bytes object

for x in range(20, 30):

bar = buf[x] # Access a data item through the pointer

# code omitted

In this instance the compiler “knows” that buf is the address of an array of bytes; it can emit code to rapidly compute the address of buf[x] at runtime. Where casts are used to convert objects to Viper native types these should be performed at the start of the function rather than in critical timing loops as the cast operation can take several microseconds. The rules for casting are as follows:

- Casting operators are currently: int, bool, uint, ptr, ptr8, ptr16 and ptr32.

- The result of a cast will be a native Viper variable.

- Arguments to a cast can be a Python object or a native Viper variable.

- If argument is a native Viper variable, then cast is a no-op (i.e. costs nothing at runtime) that just changes the type (e.g. from uint to ptr8) so that you can then store/load using this pointer.

- If the argument is a Python object and the cast is int or uint, then the Python object must be of integral type and the value of that integral object is returned.

- The argument to a bool cast must be integral type (boolean or integer); when used as a return type the viper function will return True or False objects.

- If the argument is a Python object and the cast is ptr, ptr, ptr16 or ptr32, then the Python object must either have the buffer protocol (in which case a pointer to the start of the buffer is returned) or it must be of integral type (in which case the value of that integral object is returned).

Writing to a pointer which points to a read-only object will lead to undefined behaviour.

The following example illustrates the use of a ptr16 cast to toggle pin X1 n times:

BIT0 = const(1)

@micropython.viper

def toggle_n(n: int):

odr = ptr16(stm.GPIOA + stm.GPIO_ODR)

for _ in range(n):

odr[0] ^= BIT0

A detailed technical description of the three code emitters may be found on Kickstarter here Note 1 and here Note 2

Accessing hardware directly

Note

Code examples in this section are given for the Pyboard. The techniques described however may be applied to other MicroPython ports too.

This comes into the category of more advanced programming and involves some knowledge of the target MCU. Consider the example of toggling an output pin on the Pyboard. The standard approach would be to write

mypin.value(mypin.value() ^ 1) # mypin was instantiated as an output pin

This involves the overhead of two calls to the Pin instance’s value() method. This overhead can be eliminated by performing a read/write to the relevant bit of the chip’s GPIO port output data register (odr). To facilitate this the stm module provides a set of constants providing the addresses of the relevant registers. A fast toggle of pin P4 (CPU pin A14) - corresponding to the green LED - can be performed as follows:

import machine

import stm

BIT14 = const(1 << 14)

machine.mem16[stm.GPIOA + stm.GPIO_ODR] ^= BIT14

MicroPython on microcontrollers

MicroPython is designed to be capable of running on microcontrollers. These have hardware limitations which may be unfamiliar to programmers more familiar with conventional computers. In particular the amount of RAM and nonvolatile “disk” (flash memory) storage is limited. This tutorial offers ways to make the most of the limited resources. Because MicroPython runs on controllers based on a variety of architectures, the methods presented are generic: in some cases it will be necessary to obtain detailed information from platform specific documentation.

Flash memory

On the Pyboard the simple way to address the limited capacity is to fit a micro SD card. In some cases this is impractical, either because the device does not have an SD card slot or for reasons of cost or power consumption; hence the on-chip flash must be used. The firmware including the MicroPython subsystem is stored in the onboard flash. The remaining capacity is available for use. For reasons connected with the physical architecture of the flash memory part of this capacity may be inaccessible as a filesystem. In such cases this space may be employed by incorporating user modules into a firmware build which is then flashed to the device.

There are two ways to achieve this: frozen modules and frozen bytecode. Frozen modules store the Python source with the firmware. Frozen bytecode uses the cross compiler to convert the source to bytecode which is then stored with the firmware. In either case the module may be accessed with an import statement:

import mymodule

The procedure for producing frozen modules and bytecode is platform dependent; instructions for building the firmware can be found in the README files in the relevant part of the source tree.

In general terms the steps are as follows:

- Clone the MicroPython repository.

- Acquire the (platform specific) toolchain to build the firmware.

- Build the cross compiler.

- Place the modules to be frozen in a specified directory (dependent on whether the module is to be frozen as source or as bytecode).

- Build the firmware. A specific command may be required to build frozen code of either type - see the platform documentation.

- Flash the firmware to the device.

RAM

When reducing RAM usage there are two phases to consider: compilation and execution. In addition to memory consumption, there is also an issue known as heap fragmentation. In general terms it is best to minimise the repeated creation and destruction of objects. The reason for this is covered in the section covering the heap.

Compilation phase

When a module is imported, MicroPython compiles the code to bytecode which is then executed by the MicroPython virtual machine (VM). The bytecode is stored in RAM. The compiler itself requires RAM, but this becomes available for use when the compilation has completed.

If a number of modules have already been imported the situation can arise where there is insufficient RAM to run the compiler. In this case the import statement will produce a memory exception.

If a module instantiates global objects on import it will consume RAM at the time of import, which is then unavailable for the compiler to use on subsequent imports. In general it is best to avoid code which runs on import; a better approach is to have initialisation code which is run by the application after all modules have been imported. This maximises the RAM available to the compiler.

If RAM is still insufficient to compile all modules one solution is to precompile modules. MicroPython has a cross compiler capable of compiling Python modules to bytecode (see the README in the mpy-cross directory). The resulting bytecode file has a .mpy extension; it may be copied to the filesystem and imported in the usual way. Alternatively some or all modules may be implemented as frozen bytecode: on most platforms this saves even more RAM as the bytecode is run directly from flash rather than being stored in RAM.

Execution phase

There are a number of coding techniques for reducing RAM usage.

Constants

MicroPython provides a const keyword which may be used as follows:

from micropython import const

ROWS = const(33)

_COLS = const(0x10)

a = ROWS

b = _COLS

In both instances where the constant is assigned to a variable the compiler will avoid coding a lookup to the name of the constant by substituting its literal value. This saves bytecode and hence RAM. However the ROWS value will occupy at least two machine words, one each for the key and value in the globals dictionary. The presence in the dictionary is necessary because another module might import or use it. This RAM can be saved by prepending the name with an underscore as in _COLS: this symbol is not visible outside the module so will not occupy RAM.

The argument to const() may be anything which, at compile time, evaluates to an integer e.g. 0x100 or 1 << 8. It can even include other const symbols that have already been defined, e.g. 1 << BIT.

Constant data structures

Where there is a substantial volume of constant data and the platform supports execution from Flash, RAM may be saved as follows. The data should be located in Python modules and frozen as bytecode. The data must be defined as bytes objects. The compiler ‘knows’ that bytes objects are immutable and ensures that the objects remain in flash memory rather than being copied to RAM. The ustruct module can assist in converting between bytes types and other Python built-in types.

When considering the implications of frozen bytecode, note that in Python strings, floats, bytes, integers and complex numbers are immutable. Accordingly these will be frozen into flash. Thus, in the line

mystring = "The quick brown fox"

the actual string “The quick brown fox” will reside in flash. At runtime a reference to the string is assigned to the variable mystring. The reference occupies a single machine word. In principle a long integer could be used to store constant data:

bar = 0xDEADBEEF0000DEADBEEF

As in the string example, at runtime a reference to the arbitrarily large integer is assigned to the variable bar. That reference occupies a single machine word.

It might be expected that tuples of integers could be employed for the purpose of storing constant data with minimal RAM use. With the current compiler this is ineffective (the code works, but RAM is not saved).

foo = (1, 2, 3, 4, 5, 6, 100000)

At runtime the tuple will be located in RAM. This may be subject to future improvement.

Needless object creation

There are a number of situations where objects may unwittingly be created and destroyed. This can reduce the usability of RAM through fragmentation. The following sections discuss instances of this.

String concatenation

Consider the following code fragments which aim to produce constant strings:

var = "foo" + "bar"

var1 = "foo" "bar"

var2 = """\

foo\

bar"""

Each produces the same outcome, however the first needlessly creates two string objects at runtime, allocates more RAM for concatenation before producing the third. The others perform the concatenation at compile time which is more efficient, reducing fragmentation.

Where strings must be dynamically created before being fed to a stream such as a file it will save RAM if this is done in a piecemeal fashion. Rather than creating a large string object, create a substring and feed it to the stream before dealing with the next.

The best way to create dynamic strings is by means of the string format() method:

var = "Temperature {:5.2f} Pressure {:06d}\n".format(temp, press)

Buffers

When accessing devices such as instances of UART, I2C and SPI interfaces, using pre-allocated buffers avoids the creation of needless objects. Consider these two loops:

while True:

var = spi.read(100)

# process data

buf = bytearray(100)

while True:

spi.readinto(buf)

# process data in buf

The first creates a buffer on each pass whereas the second re-uses a pre-allocated buffer; this is both faster and more efficient in terms of memory fragmentation.

Bytes are smaller than ints

On most platforms an integer consumes four bytes. Consider the two calls to the function foo():

def foo(bar):

for x in bar:

print(x)

foo((1, 2, 0xff))

foo(b'\1\2\xff')

In the first call a tuple of integers is created in RAM. The second efficiently creates a bytes object consuming the minimum amount of RAM. If the module were frozen as bytecode, the bytes object would reside in flash.

Strings Versus Bytes

Python3 introduced Unicode support. This introduced a distinction between a string and an array of bytes. MicroPython ensures that Unicode strings take no additional space so long as all characters in the string are ASCII (i.e. have a value < 126). If values in the full 8-bit range are required bytes and bytearray objects can be used to ensure that no additional space will be required. Note that most string methods (e.g. str.strip()) apply also to bytes instances so the process of eliminating Unicode can be painless.

s = 'the quick brown fox'   # A string instance

b = b'the quick brown fox'  # A bytes instance

Where it is necessary to convert between strings and bytes the str.encode() and the bytes.decode() methods can be used. Note that both strings and bytes are immutable. Any operation which takes as input such an object and produces another implies at least one RAM allocation to produce the result. In the second line below a new bytes object is allocated. This would also occur if foo were a string.

foo = b'   empty whitespace'

foo = foo.lstrip()

Runtime compiler execution

The Python funcitons eval and exec invoke the compiler at runtime, which requires significant amounts of RAM. Note that the pickle library from micropython-lib employs exec. It may be more RAM efficient to use the ujson library for object serialisation.

Storing strings in flash

Python strings are immutable hence have the potential to be stored in read only memory. The compiler can place in flash strings defined in Python code. As with frozen modules it is necessary to have a copy of the source tree on the PC and the toolchain to build the firmware. The procedure will work even if the modules have not been fully debugged, so long as they can be imported and run.

After importing the modules, execute:

micropython.qstr_info(1)

Then copy and paste all the Q(xxx) lines into a text editor. Check for and remove lines which are obviously invalid. Open the file qstrdefsport.h which will be found in ports/stm32 (or the equivalent directory for the architecture in use). Copy and paste the corrected lines at the end of the file. Save the file, rebuild and flash the firmware. The outcome can be checked by importing the modules and again issuing:

micropython.qstr_info(1)

The Q(xxx) lines should be gone.

The heap

When a running program instantiates an object the necessary RAM is allocated from a fixed size pool known as the heap. When the object goes out of scope (in other words becomes inaccessible to code) the redundant object is known as “garbage”. A process known as “garbage collection” (GC) reclaims that memory, returning it to the free heap. This process runs automatically, however it can be invoked directly by issuing gc.collect().

The discourse on this is somewhat involved. For a ‘quick fix’ issue the following periodically:

gc.collect()

gc.threshold(gc.mem_free() // 4 + gc.mem_alloc())

Fragmentation

Say a program creates an object foo, then an object bar. Subsequently foo goes out of scope but bar remains. The RAM used by foo will be reclaimed by GC. However if bar was allocated to a higher address, the RAM reclaimed from foo will only be of use for objects no bigger than foo. In a complex or long running program the heap can become fragmented: despite there being a substantial amount of RAM available, there is insufficient contiguous space to allocate a particular object, and the program fails with a memory error.

The techniques outlined above aim to minimise this. Where large permanent buffers or other objects are required it is best to instantiate these early in the process of program execution before fragmentation can occur. Further improvements may be made by monitoring the state of the heap and by controlling GC; these are outlined below.

Reporting

A number of library functions are available to report on memory allocation and to control GC. These are to be found in the gc and micropython modules. The following example may be pasted at the REPL (ctrl e to enter paste mode, ctrl d to run it).

import gc

import micropython

gc.collect()

micropython.mem_info()

print('-----------------------------')

print('Initial free: {} allocated: {}'.format(gc.mem_free(), gc.mem_alloc()))

def func():

a = bytearray(10000)

gc.collect()

print('Func definition: {} allocated: {}'.format(gc.mem_free(), gc.mem_alloc()))

func()

print('Func run free: {} allocated: {}'.format(gc.mem_free(), gc.mem_alloc()))

gc.collect()

print('Garbage collect free: {} allocated: {}'.format(gc.mem_free(), gc.mem_alloc()))

print('-----------------------------')

micropython.mem_info(1)

Methods employed above:

- gc.collect() Force a garbage collection. See footnote.

- micropython.mem_info() Print a summary of RAM utilisation.

- gc.mem_free() Return the free heap size in bytes.

- gc.mem_alloc() Return the number of bytes currently allocated.

- micropython.mem_info(1) Print a table of heap utilisation (detailed below).

The numbers produced are dependent on the platform, but it can be seen that declaring the function uses a small amount of RAM in the form of bytecode emitted by the compiler (the RAM used by the compiler has been reclaimed). Running the function uses over 10KiB, but on return a is garbage because it is out of scope and cannot be referenced. The final gc.collect() recovers that memory.

The final output produced by micropython.mem_info(1) will vary in detail but may be interpreted as follows:

| Symbol | Meaning |
| --- | --- |
| . | free block |
| h | head block |
| = | tail block |
| m | marked head block |
| T | tuple |
| L | list |
| D | dict |
| F | float |
| B | byte code |
| M | module |

Each letter represents a single block of memory, a block being 16 bytes. So each line of the heap dump represents 0x400 bytes or 1KiB of RAM.

Control of garbage collection

A GC can be demanded at any time by issuing gc.collect(). It is advantageous to do this at intervals, firstly to pre-empt fragmentation and secondly for performance. A GC can take several milliseconds but is quicker when there is little work to do (about 1ms on the Pyboard). An explicit call can minimise that delay while ensuring it occurs at points in the program when it is acceptable.

Automatic GC is provoked under the following circumstances. When an attempt at allocation fails, a GC is performed and the allocation re-tried. Only if this fails is an exception raised. Secondly an automatic GC will be triggered if the amount of free RAM falls below a threshold. This threshold can be adapted as execution progresses:

gc.collect()

gc.threshold(gc.mem_free() // 4 + gc.mem_alloc())

This will provoke a GC when more than 25% of the currently free heap becomes occupied.

In general modules should instantiate data objects at runtime using constructors or other initialisation functions. The reason is that if this occurs on initialisation the compiler may be starved of RAM when subsequent modules are imported. If modules do instantiate data on import then gc.collect() issued after the import will ameliorate the problem.

String operations

MicroPython handles strings in an efficient manner and understanding this can help in designing applications to run on microcontrollers. When a module is compiled, strings which occur multiple times are stored once only, a process known as string interning. In MicroPython an interned string is known as a qstr. In a module imported normally that single instance will be located in RAM, but as described above, in modules frozen as bytecode it will be located in flash.

String comparisons are also performed efficiently using hashing rather than character by character. The penalty for using strings rather than integers may hence be small both in terms of performance and RAM usage - a fact which may come as a surprise to C programmers.

Postscript

MicroPython passes, returns and (by default) copies objects by reference. A reference occupies a single machine word so these processes are efficient in RAM usage and speed.

Where variables are required whose size is neither a byte nor a machine word there are standard libraries which can assist in storing these efficiently and in performing conversions. See the array, ustruct and uctypes modules.

Footnote: gc.collect() return value

On Unix and Windows platforms the gc.collect() method returns an integer which signifies the number of distinct memory regions that were reclaimed in the collection (more precisely, the number of heads that were turned into frees). For efficiency reasons bare metal ports do not return this value.

Distribution packages, package management, and deploying applications

Just as the “big” Python, MicroPython supports creation of “third party” packages, distributing them, and easily installing them in each user’s environment. This chapter discusses how these actions are achieved. Some familiarity with Python packaging is recommended.

Overview

Steps below represent a high-level workflow when creating and consuming packages:

- Python modules and packages are turned into distribution package archives, and published at the Python Package Index (PyPI).

- upip package manager can be used to install a distribution package on a MicroPython port with networking capabilities (for example, on the Unix port).

- For ports without networking capabilities, an “installation image” can be prepared on the Unix port, and transferred to a device by suitable means.

- For low-memory ports, the installation image can be frozen as the bytecode into MicroPython executable, thus minimizing the memory storage overheads.

The sections below describe this process in details.

Distribution packages

Python modules and packages can be packaged into archives suitable for transfer between systems, storing at the well-known location (PyPI), and downloading on demand for deployment. These archives are known as distribution packages (to differentiate them from Python packages (means to organize Python source code)).

The MicroPython distribution package format is a well-known tar.gz format, with some adaptations however. The Gzip compressor, used as an external wrapper for TAR archives, by default uses 32KB dictionary size, which means that to uncompress a compressed stream, 32KB of contiguous memory needs to be allocated. This requirement may be not satisfiable on low-memory devices, which may have total memory available less than that amount, and even if not, a contiguous block like that may be hard to allocate due to memory fragmentation. To accommodate these constraints, MicroPython distribution packages use Gzip compression with the dictionary size of 4K, which should be a suitable compromise with still achieving some compression while being able to uncompressed even by the smallest devices.

Besides the small compression dictionary size, MicroPython distribution packages also have other optimizations, like removing any files from the archive which aren’t used by the installation process. In particular, upip package manager doesn’t execute setup.py during installation (see below), and thus that file is not included in the archive.

At the same time, these optimizations make MicroPython distribution packages not compatible with CPython‘s package manager, pip. This isn’t considered a big problem, because:

- Packages can be installed with upip, and then can be used with CPython (if they are compatible with it).

- In the other direction, majority of CPython packages would be incompatible with MicroPython by various reasons, first of all, the reliance on features not implemented by MicroPython.

Summing up, the MicroPython distribution package archives are highly optimized for MicroPython’s target environments, which are highly resource constrained devices.

upip package manager

MicroPython distribution packages are intended to be installed using the upip package manager. upip is a Python application which is usually distributed (as frozen bytecode) with network-enabled MicroPython ports. At the very least, upip is available in the MicroPython Unix port.

On any MicroPython port providing upip, it can be accessed as following:

import upip

upip.help()

upip.install(package_or_package_list, [path])

Where package_or_package_list is the name of a distribution package to install, or a list of such names to install multiple packages. Optional path parameter specifies filesystem location to install under and defaults to the standard library location (see below).

An example of installing a specific package and then using it:

>>> import upip

>>> upip.install("micropython-pystone_lowmem")

[...]

>>> import pystone_lowmem

>>> pystone_lowmem.main()

Note that the name of Python package and the name of distribution package for it in general don’t have to match, and oftentimes they don’t. This is because PyPI provides a central package repository for all different Python implementations and versions, and thus distribution package names may need to be namespaced for a particular implementation. For example, all packages from micropython-lib follow this naming convention: for a Python module or package named foo, the distribution package name is micropython-foo.

For the ports which run MicroPython executable from the OS command prompts (like the Unix port), upip can be (and indeed, usually is) run from the command line instead of MicroPython’s own REPL. The commands which corresponds to the example above are:

micropython -m upip -h

micropython -m upip install [-p <path>] <packages>...

micropython -m upip install micropython-pystone_lowmem

[TODO: Describe installation path.]

Cross-installing packages

For MicroPython ports without native networking capabilities, the recommend process is “cross-installing” them into a “directory image” using the MicroPython Unix port, and then transferring this image to a device by suitable means.

Installing to a directory image involves using -p switch to upip:

micropython -m upip install -p install_dir micropython-pystone_lowmem

After this command, the package content (and contents of every dependency packages) will be available in the install_dir/ subdirectory. You would need to transfer contents of this directory (without the install_dir/ prefix) to the device, at the suitable location, where it can be found by the Python import statement (see discussion of the upip installation path above).

Cross-installing packages with freezing

For the low-memory MicroPython ports, the process described in the previous section does not provide the most efficient resource usage,because the packages are installed in the source form, so need to be compiled to the bytecome on each import. This compilation requires RAM, and the resulting bytecode is also stored in RAM, reducing its amount available for storing application data. Moreover, the process above requires presence of the filesystem on a device, and the most resource-constrained devices may not even have it.

The bytecode freezing is a process which resolves all the issues mentioned above:

- The source code is pre-compiled into bytecode and store as such.

- The bytecode is stored in ROM, not RAM.

- Filesystem is not required for frozen packages.

Using frozen bytecode requires building the executable (firmware) for a given MicroPython port from the C source code. Consequently, the process is:

- Follow the instructions for a particular port on setting up a toolchain and building the port. For example, for ESP8266 port, study instructions in ports/esp8266/README.md and follow them. Make sure you can build the port and deploy the resulting executable/firmware successfully before proceeding to the next steps.

- Build MicroPython Unix port and make sure it is in your PATH and you can execute micropython.

- Change to port’s directory (e.g. ports/esp8266/ for ESP8266).

- Run make clean-frozen. This step cleans up any previous modules which were installed for freezing (consequently, you need to skip this step to add additional modules, instead of starting from scratch).

- Run micropython -m upip install -p modules <packages>... to install packages you want to freeze.

- Run make clean.

- Run make.

After this, you should have the executable/firmware with modules as the bytecode inside, which you can deploy the usual way.

Few notes:

- Step 5 in the sequence above assumes that the distribution package is available from PyPI. If that is not the case, you would need to copy Python source files manually to modules/ subdirectory of the port port directory. (Note that upip does not support installing from e.g. version control repositories).

- The firmware for baremetal devices usually has size restrictions, so adding too many frozen modules may overflow it. Usually, you would get a linking error if this happens. However, in some cases, an image may be produced, which is not runnable on a device. Such cases are in general bugs, and should be reported and further investigated. If you face such a situation, as an initial step, you may want to decrease the amount of frozen modules included.

Creating distribution packages

Distribution packages for MicroPython are created in the same manner as for CPython or any other Python implementation, see references at the end of chapter. Setuptools (instead of distutils) should be used, because distutils do not support dependencies and other features. “Source distribution” (sdist) format is used for packaging. The post-processing discussed above, (and pre-processing discussed in the following section) is achieved by using custom sdist command for setuptools. Thus, packaging steps remain the same as for the standard setuptools, the user just needs to override sdist command implementation by passing the appropriate argument to setup() call:

from setuptools import setup

import sdist_upip

setup(

...,

cmdclass={'sdist': sdist_upip.sdist}

)

The sdist_upip.py module as referenced above can be found in micropython-lib:

https://github.com/micropython/micropython-lib/blob/master/sdist_upip.py

Application resources

A complete application, besides the source code, oftentimes also consists of data files, e.g. web page templates, game images, etc. It’s clear how to deal with those when application is installed manually - you just put those data files in the filesystem at some location and use the normal file access functions.

The situation is different when deploying applications from packages - this is more advanced, streamlined and flexible way, but also requires more advanced approach to accessing data files. This approach is treating the data files as “resources”, and abstracting away access to them.

Python supports resource access using its “setuptools” library, using pkg_resources module. MicroPython, following its usual approach, implements subset of the functionality of that module, specifically pkg_resources.resource_stream(package, resource) function. The idea is that an application calls this function, passing a resource identifier, which is a relative path to data file within the specified package (usually top-level application package). It returns a stream object which can be used to access resource contents. Thus, the resource_stream() emulates interface of the standard open() function.

Implementation-wise, resource_stream() uses file operations underlyingly, if distribution package is install in the filesystem. However, it also supports functioning without the underlying filesystem, e.g. if the package is frozen as the bytecode. This however requires an extra intermediate step when packaging application - creation of “Python resource module”.

The idea of this module is to convert binary data to a Python bytes object, and put it into the dictionary, indexed by the resource name. This conversion is done automatically using overridden sdist command described in the previous section.

Let’s trace the complete process using the following example. Suppose your application has the following structure:

my_app/

__main__.py

utils.py

data/

page.html

image.png

__main__.py and utils.py should access resources using the following calls:

import pkg_resources

pkg_resources.resource_stream(__name__, "data/page.html")

pkg_resources.resource_stream(__name__, "data/image.png")

You can develop and debug using the MicroPython Unix port as usual. When time comes to make a distribution package out of it, just use overridden “sdist” command from sdist_upip.py module as described in the previous section.

This will create a Python resource module named R.py, based on the files declared in MANIFEST or MANIFEST.in files (any non-.py file will be considered a resource and added to R.py) - before proceeding with the normal packaging steps.

Prepared like this, your application will work both when deployed to filesystem and as frozen bytecode.

If you would like to debug R.py creation, you can run:

python3 setup.py sdist --manifest-only

Alternatively, you can use tools/mpy_bin2res.py script from the MicroPython distribution, in which can you will need to pass paths to all resource files:

mpy_bin2res.py data/page.html data/image.png

References

Python Packaging User Guide: https://packaging.python.org/

Setuptools documentation: https://setuptools.readthedocs.io/

Distutils documentation: https://docs.python.org/3/library/distutils.html

Inline assembler for Thumb2 architectures

This document assumes some familiarity with assembly language programming and should be read after studying the tutorial. For a detailed description of the instruction set consult the Architecture Reference Manual detailed below. The inline assembler supports a subset of the ARM Thumb-2 instruction set described here. The syntax tries to be as close as possible to that defined in the above ARM manual, converted to Python function calls.

Instructions operate on 32 bit signed integer data except where stated otherwise. Most supported instructions operate on registers R0-R7 only: where R8-R15 are supported this is stated. Registers R8-R12 must be restored to their initial value before return from a function. Registers R13-R15 constitute the Link Register, Stack Pointer and Program Counter respectively.

Document conventions

Where possible the behaviour of each instruction is described in Python, for example

- add(Rd, Rn, Rm) Rd = Rn + Rm

This enables the effect of instructions to be demonstrated in Python. In certain case this is impossible because Python doesn’t support concepts such as indirection. The pseudocode employed in such cases is described on the relevant page.

Instruction categories

The following sections details the subset of the ARM Thumb-2 instruction set supported by MicroPython.

- Register move instructions

- Load register from memory

- Store register to memory

- Logical & bitwise instructions

- Arithmetic instructions

- Comparison instructions

- Branch instructions

- Stack push and pop

- Miscellaneous instructions

- Floating point instructions

- Assembler directives

Usage examples

These sections provide further code examples and hints on the use of the assembler.

- Hints and tips

References

- Assembler Tutorial

- Wiki hints and tips

- uPy Inline Assembler source-code, emitinlinethumb.c

- ARM Thumb2 Instruction Set Quick Reference Card

- RM0090 Reference Manual

ARM v7-M Architecture Reference Manual (Available on the ARM site after a simple registration procedure. Also available on academic sites but beware of out of date versions.)

Working with file systems

Contents

- Working with filesystems

- VFS

- Block devices

  - Built-in block devices

    - STM32 / Pyboard

    - ESP8266

    - ESP32

  - Custom block devices

- Filesystems

- FAT

- Littlefs

- Hybrid (STM32)

- Hybrid (ESP32)

This tutorial describes how MicroPython provides an on-device filesystem, allowing standard Python file I/O methods to be used with persistent storage. MicroPython automatically creates a default configuration and auto-detects the primary filesystem, so this tutorial will be mostly useful if you want to modify the partitioning, filesystem type, or use custom block devices. The filesystem is typically backed by internal flash memory on the device, but can also use external flash, RAM, or a custom block device. On some ports (e.g. STM32), the filesystem may also be available over USB MSC to a host PC. The pyboard.py tool also provides a way for the host PC to access to the filesystem on all ports.

Note: This is mainly for use on bare-metal ports like STM32 and ESP32. On ports with an operating system (e.g. the Unix port) the filesystem is provided by the host OS.

VFS

MicroPython implements a Unix-like Virtual File System (VFS) layer. All mounted filesystems are combined into a single virtual filesystem, starting at the root /. Filesystems are mounted into directories in this structure, and at startup the working directory is changed to where the primary filesystem is mounted.

On STM32 / Pyboard, the internal flash is mounted at /flash, and optionally the SDCard at /sd. On ESP8266/ESP32, the primary filesystem is mounted at /.

Block devices

A block device is an instance of a class that implements the uos.AbstractBlockDev protocol.

Built-in block devices

Ports provide built-in block devices to access their primary flash.

On power-on, MicroPython will attempt to detect the filesystem on the default flash and configure and mount it automatically. If no filesystem is found, MicroPython will attempt to create a FAT filesystem spanning the entire flash. Ports can also provide a mechanism to “factory reset” the primary flash, usually by some combination of button presses at power on.

STM32 / Pyboard

The pyb.Flash class provides access to the internal flash. On some boards which have larger external flash (e.g. Pyboard D), it will use that instead. The start kwarg should always be specified, i.e. pyb.Flash(start=0).

Note: For backwards compatibility, when constructed with no arguments (i.e. pyb.Flash()), it only implements the simple block interface and reflects the virtual device presented to USB MSC (i.e. it includes a virtual partition table at the start).

ESP8266

The internal flash is exposed as a block device object which is created in the flashbdev module on start up. This object is by default added as a global variable so it can usually be accessed simply as bdev. This implements the extended interface.

ESP32

The esp32.Partition class implements a block device for partitions defined for the board. Like ESP8266, there is a global variable bdev which points to the default partition. This implements the extended interface.

Custom block devices

The following class implements a simple block device that stores its data in RAM using a bytearray:

class RAMBlockDev:

def __init__(self, block_size, num_blocks):

self.block_size = block_size

self.data = bytearray(block_size * num_blocks)

def readblocks(self, block_num, buf):

for i in range(len(buf)):

buf[i] = self.data[block_num * self.block_size + i]

def writeblocks(self, block_num, buf):

for i in range(len(buf)):

self.data[block_num * self.block_size + i] = buf[i]

def ioctl(self, op, arg):

if op == 4: # get number of blocks

return len(self.data) // self.block_size

if op == 5: # get block size

return self.block_size

It can be used as follows:

import os

bdev = RAMBlockDev(512, 50)

os.VfsFat.mkfs(bdev)

os.mount(bdev, '/ramdisk')

An example of a block device that supports both the simple and extended interface (i.e. both signatures and behaviours of the uos.AbstractBlockDev.readblocks() and uos.AbstractBlockDev.writeblocks() methods) is:

class RAMBlockDev:

def __init__(self, block_size, num_blocks):

self.block_size = block_size

self.data = bytearray(block_size * num_blocks)

def readblocks(self, block_num, buf, offset=0):

addr = block_num * self.block_size + offset

for i in range(len(buf)):

buf[i] = self.data[addr + i]

def writeblocks(self, block_num, buf, offset=None):

if offset is None:

# do erase, then write

for i in range(len(buf) // self.block_size):

self.ioctl(6, block_num + i)

offset = 0

addr = block_num * self.block_size + offset

for i in range(len(buf)):

self.data[addr + i] = buf[i]

def ioctl(self, op, arg):

if op == 4: # block count

return len(self.data) // self.block_size

if op == 5: # block size

return self.block_size

if op == 6: # block erase

return 0

As it supports the extended interface, it can be used with littlefs:

import os

bdev = RAMBlockDev(512, 50)

os.VfsLfs2.mkfs(bdev)

os.mount(bdev, '/ramdisk')

Once mounted, the filesystem (regardless of its type) can be used as it normally would be used from Python code, for example:

with open('/ramdisk/hello.txt', 'w') as f:

f.write('Hello world')

print(open('/ramdisk/hello.txt').read())

Filesystems

MicroPython ports can provide implementations of FAT, littlefs v1 and littlefs v2.

The following table shows which filesystems are included in the firmware by default for given port/board combinations, however they can be optionally enabled in a custom firmware build.

| Board | FAT | littlefs v1 | littlefs v2 |
| --- | --- | --- | --- |
| pyboard 1.0, 1.1, D | Yes | No | Yes |
| Other STM32 | Yes | No | No |
| ESP8266 | Yes | No | No |
| ESP32 | Yes | No | Yes |

FAT

The main advantage of the FAT filesystem is that it can be accessed over USB MSC on supported boards (e.g. STM32) without any additional drivers required on the host PC.

However, FAT is not tolerant to power failure during writes and this can lead to filesystem corruption. For applications that do not require USB MSC, it is recommended to use littlefs instead.

To format the entire flash using FAT:

# ESP8266 and ESP32

import os

os.umount('/')

os.VfsFat.mkfs(bdev)

os.mount(bdev, '/')

# STM32

import os, pyb

os.umount('/flash')

os.VfsFat.mkfs(pyb.Flash(start=0))

os.mount(pyb.Flash(start=0), '/flash')

os.chdir('/flash')

Littlefs

Littlefs is a filesystem designed for flash-based devices, and is much more resistant to filesystem corruption.

There are reports of littlefs v1 and v2 failing in certain situations, for details see littlefs issue 347 and littlefs issue 295.

Note: It can be still be accessed over USB MSC using the littlefs FUSE driver. Note that you must use the -b=4096 option to override the block size.

To format the entire flash using littlefs v2:

# ESP8266 and ESP32

import os

os.umount('/')

os.VfsLfs2.mkfs(bdev)

os.mount(bdev, '/')

# STM32

import os, pyb

os.umount('/flash')

os.VfsLfs2.mkfs(pyb.Flash(start=0))

os.mount(pyb.Flash(start=0), '/flash')

os.chdir('/flash')

Hybrid (STM32)

By using the start and len kwargs to pyb.Flash, you can create block devices spanning a subset of the flash device.

For example, to configure the first 256kiB as FAT (and available over USB MSC), and the remainder as littlefs:

import os, pyb

os.umount('/flash')

p1 = pyb.Flash(start=0, len=256*1024)

p2 = pyb.Flash(start=256*1024)

os.VfsFat.mkfs(p1)

os.VfsLfs2.mkfs(p2)

os.mount(p1, '/flash')

os.mount(p2, '/data')

os.chdir('/flash')

This might be useful to make your Python files, configuration and other rarely-modified content available over USB MSC, but allowing for frequently changing application data to reside on littlefs with better resilience to power failure, etc.

The partition at offset 0 will be mounted automatically (and the filesystem type automatically detected), but you can add:

import os, pyb

p2 = pyb.Flash(start=256*1024)

os.mount(p2, '/data')

to boot.py to mount the data partition.

Hybrid (ESP32)

On ESP32, if you build custom firmware, you can modify partitions.csv to define an arbitrary partition layout.

At boot, the partition named “vfs” will be mounted at / by default, but any additional partitions can be mounted in your boot.py using:

import esp32, os

p = esp32.Partition.find(esp32.Partition.TYPE_DATA, label='foo')

os.mount(p, '/foo')

The pyboard.py tool

This is a standalone Python tool that runs on your PC that provides a way to:

- Quickly run a Python script or command on a MicroPython device. This is useful while developing MicroPython programs to quickly test code without needing to copy files to/from the device.

- Access the filesystem on a device. This allows you to deploy your code to the device (even if the board doesn’t support USB MSC).

Despite the name, pyboard.py works on all MicroPython ports that support the raw REPL (including STM32, ESP32, ESP8266, NRF).

You can download the latest version from GitHub. The only dependency is the pyserial library which can be installed from PiPy or your system package manager.

Running pyboard.py --help gives the following output:

usage: pyboard [-h] [--device DEVICE] [-b BAUDRATE] [-u USER]

[-p PASSWORD] [-c COMMAND] [-w WAIT] [--follow] [-f]

[files [files ...]]

Run scripts on the pyboard.

positional arguments:

files                 input files

optional arguments:

-h, --help            show this help message and exit

--device DEVICE       the serial device or the IP address of the

pyboard

-b BAUDRATE, --baudrate BAUDRATE

the baud rate of the serial device

-u USER, --user USER  the telnet login username

-p PASSWORD, --password PASSWORD

the telnet login password

-c COMMAND, --command COMMAND

program passed in as string

-w WAIT, --wait WAIT  seconds to wait for USB connected board to become

available

--follow              follow the output after running the scripts

[default if no scripts given]

-f, --filesystem      perform a filesystem action

Running a command on the device

This is useful for testing short snippets of code, or to script an interaction with the device.:

$ pyboard.py --device /dev/ttyACM0 -c 'print(1+1)'

2

Running a script on the device

If you have a script, app.py that you want to run on a device, then use:

$ pyboard.py --device /dev/ttyACM0 app.py

Note that this doesn’t actually copy app.py to the device’s filesystem, it just loads the code into RAM and executes it. Any output generated by the program will be displayed.

If the program app.py does not finish then you’ll need to stop pyboard.py, eg with Ctrl-C. The program app.py will still continue to run on the MicroPython device.

Filesystem access

Using the -f flag, the following filesystem operations are supported:

- cp src [src...] dest Copy files to/from the device.

- cat path Print the contents of a file on the device.

- ls [path] List contents of a directory (defaults to current working directory).

- rm path Remove a file.

- mkdir path Create a directory.

- rmdir path Remove a directory.

The cp command uses a ssh-like convention for referring to local and remote files. Any path starting with a : will be interpreted as on the device, otherwise it will be local. So:

$ pyboard.py --device /dev/ttyACM0 -f cp main.py :main.py

will copy main.py from the current directory on the PC to a file named main.py on the device. The filename can be omitted, e.g.:

$ pyboard.py --device /dev/ttyACM0 -f cp main.py :

is equivalent to the above.

Some more examples:

# Copy main.py from the device to the local PC.

$ pyboard.py --device /dev/ttyACM0 -f cp :main.py main.py

# Same, but using . instead.

$ pyboard.py --device /dev/ttyACM0 -f cp :main.py .

# Copy three files to the device, keeping their names

# and paths (note: `lib` must exist on the device)

$ pyboard.py --device /dev/ttyACM0 -f cp main.py app.py lib/foo.py :

# Remove a file from the device.

$ pyboard.py --device /dev/ttyACM0 -f rm util.py

# Print the contents of a file on the device.

$ pyboard.py --device /dev/ttyACM0 -f cat boot.py

...contents of boot.py...

Using the pyboard library

You can also use pyboard.py as a library for scripting interactions with a MicroPython board.

import pyboard

pyb = pyboard.Pyboard('/dev/ttyACM0', 115200)

pyb.enter_raw_repl()

ret = pyb.exec('print(1+1)')

print(ret)

pyb.exit_raw_repl()

MicroPython libraries

Warning

Important summary of this section

- MicroPython implements a subset of Python functionality for each module.

- To ease extensibility, MicroPython versions of standard Python modules usually have u (“micro”) prefix.

- Any particular MicroPython variant or port may miss any feature/function described in this general documentation (due to resource constraints or other limitations).

This chapter describes modules (function and class libraries) which are built into MicroPython.

There are a few categories of such modules:

- Modules which implement a subset of standard Python functionality and are not intended to be extended by the user.

- Modules which implement a subset of Python functionality, with a provision for extension by the user (via Python code).

- Modules which implement MicroPython extensions to the Python standard libraries.

- Modules specific to a particular MicroPython port and thus not portable.

Note about the availability of the modules and their contents: This documentation in general aspires to describe all modules and functions/classes which are implemented in MicroPython project. However, MicroPython is highly configurable, and each port to a particular board/embedded system makes available only a subset of MicroPython libraries. For officially supported ports, there is an effort to either filter out non-applicable items, or mark individual descriptions with “Availability:” clauses describing which ports provide a given feature.

With that in mind, please still be warned that some functions/classes in a module (or even the entire module) described in this documentation may be unavailable in a particular build of MicroPython on a particular system. The best place to find general information of the availability/non-availability of a particular feature is the “General Information” section which contains information pertaining to a specific MicroPython port.

On some ports you are able to discover the available, built-in libraries that can be imported by entering the following at the REPL:

help('modules')

Beyond the built-in libraries described in this documentation, many more modules from the Python standard library, as well as further MicroPython extensions to it, can be found in micropython-lib.

Python standard libraries and micro-libraries

The following standard Python libraries have been “micro-ified” to fit in with the philosophy of MicroPython. They provide the core functionality of that module and are intended to be a drop-in replacement for the standard Python library. Some modules below use a standard Python name, but prefixed with “u”, e.g. ujson instead of json. This is to signify that such a module is micro-library, i.e. implements only a subset of CPython module functionality. By naming them differently, a user has a choice to write a Python-level module to extend functionality for better compatibility with CPython (indeed, this is what done by the micropython-lib project mentioned above).

On some embedded platforms, where it may be cumbersome to add Python-level wrapper modules to achieve naming compatibility with CPython, micro-modules are available both by their u-name, and also by their non-u-name. The non-u-name can be overridden by a file of that name in your library path (sys.path). For example, import json will first search for a file json.py (or package directory json) and load that module if it is found. If nothing is found, it will fallback to loading the built-in ujson module.

- Builtin functions and exceptions

- cmath – mathematical functions for complex numbers

- gc – control the garbage collector

- math – mathematical functions

- sys – system specific functions

- uarray – arrays of numeric data

- ubinascii – binary/ASCII conversions

- ucollections – collection and container types

- uerrno – system error codes

- uhashlib – hashing algorithms

- uheapq – heap queue algorithm

- uio – input/output streams

- ujson – JSON encoding and decoding

- uos – basic “operating system” services

- ure – simple regular expressions

- uselect – wait for events on a set of streams

- usocket – socket module

- ussl – SSL/TLS module

- ustruct – pack and unpack primitive data types

- utime – time related functions

- uzlib – zlib decompression

- _thread – multithreading support

MicroPython-specific libraries

Functionality specific to the MicroPython implementation is available in the following libraries.

- btree – simple BTree database

- framebuf — frame buffer manipulation

- machine — functions related to the hardware

- micropython – access and control MicroPython internals

- network — network configuration

- ubluetooth — low-level Bluetooth

- ucryptolib – cryptographic ciphers

- uctypes – access binary data in a structured way

Port-specific libraries

In some cases the following port/board-specific libraries have functions or classes similar to those in the machine library. Where this occurs, the entry in the port specific library exposes hardware functionality unique to that platform.

To write portable code use functions and classes from the machine module. To access platform-specific hardware use the appropriate library, e.g. pyb in the case of the Pyboard.

Libraries specific to the pyboard

The following libraries are specific to the pyboard.

- pyb — functions related to the board

- Time related functions

- Reset related functions

- Interrupt related functions

- Power related functions

- Miscellaneous functions

- Classes

- lcd160cr — control of LCD160CR display

- class LCD160CR

- Constructors

- Static methods

- Instance members

- Setup commands

- Pixel access methods

- Drawing text

- Drawing primitive shapes

- Touch screen methods

- Advanced commands

- Constants

Libraries specific to the WiPy

The following libraries and classes are specific to the WiPy.

- wipy – WiPy specific features

- Functions

- class ADCWiPy – analog to digital conversion

- Constructors

- Methods

- class ADCChannel — read analog values from internal or external sources

- class TimerWiPy – control hardware timers

- Constructors

- Methods

- class TimerChannel — setup a channel for a timer

- Methods

- Constants

Libraries specific to the ESP8266 and ESP32

The following libraries are specific to the ESP8266 and ESP32.

- esp — functions related to the ESP8266 and ESP32

- Functions

- esp32 — functionality specific to the ESP32

- Functions

- Flash partitions

- RMT

- Ultra-Low-Power co-processor

- Constants

MicroPython differences from CPython

The operations listed in this section produce conflicting results in MicroPython when compared to standard Python. MicroPython implements Python 3.4 and some select features of Python 3.5.

- Syntax

- Spaces

- Unicode

- Core language

- Classes

- Functions

- Generator

- Runtime

- import

- Builtin types

- Exception

- bytearray

- bytes

- dict

- float

- int

- list

- str

- tuple

- Modules

- array

- builtins

- deque

- json

- struct

- sys

Syntax

Generated Thu 09 Jan 2020 12:08:52 UTC

Spaces

uPy requires spaces between literal numbers and keywords, CPy doesn’t

Sample code:

try:

print(eval('1and 0'))

except SyntaxError:

print('Should have worked')

try:

print(eval('1or 0'))

except SyntaxError:

print('Should have worked')

try:

print(eval('1if 1else 0'))

except SyntaxError:

print('Should have worked')

| CPy output: | uPy output: |
| --- | --- |
| 0 1 1 | Should have worked Should have worked Should have worked |

Unicode

Unicode name escapes are not implemented

Sample code:

print("\N{LATIN SMALL LETTER A}")

| CPy output: | uPy output: |
| --- | --- |
| a | NotImplementedError: unicode name escapes |

Core language

Generated Thu 09 Jan 2020 12:08:52 UTC

Classes

Special method __del__ not implemented for user-defined classes

Sample code:

import gc

class Foo():

def __del__(self):

print('__del__')

f = Foo()

del f

gc.collect()

| CPy output: | uPy output: |
| --- | --- |
| __del__ |  |

Method Resolution Order (MRO) is not compliant with CPython

Cause: Depth first non-exhaustive method resolution order

Workaround: Avoid complex class hierarchies with multiple inheritance and complex method overrides. Keep in mind that many languages don’t support multiple inheritance at all.

Sample code:

class Foo:

def __str__(self):

return "Foo"

class C(tuple, Foo):

pass

t = C((1, 2, 3))

print(t)

| CPy output: | uPy output: |
| --- | --- |
| Foo | (1, 2, 3) |

When inheriting from multiple classes super() only calls one class

Cause: See Method Resolution Order (MRO) is not compliant with CPython

Workaround: See Method Resolution Order (MRO) is not compliant with CPython

Sample code:

class A:

def __init__(self):

print("A.__init__")

class B(A):

def __init__(self):

print("B.__init__")

super().__init__()

class C(A):

def __init__(self):

print("C.__init__")

super().__init__()

class D(B,C):

def __init__(self):

print("D.__init__")

super().__init__()

D()

| CPy output: | uPy output: |
| --- | --- |
| D.__init__ B.__init__ C.__init__ A.__init__ | D.__init__ B.__init__ A.__init__ |

Calling super() getter property in subclass will return a property object, not the value

Sample code:

class A:

@property

def p(self):

return {"a":10}

class AA(A):

@property

def p(self):

return super().p

a = AA()

print(a.p)

| CPy output: | uPy output: |
| --- | --- |
| {'a': 10} | <property> |

Functions

Error messages for methods may display unexpected argument counts

Cause: MicroPython counts “self” as an argument.

Workaround: Interpret error messages with the information above in mind.

Sample code:

try:

[].append()

except Exception as e:

print(e)

| CPy output: | uPy output: |
| --- | --- |
| append() takes exactly one argument (0 given) | function takes 2 positional arguments but 1 were given |

User-defined attributes for functions are not supported

Cause: MicroPython is highly optimized for memory usage.

Workaround: Use external dictionary, e.g. FUNC_X[f] = 0.

Sample code:

def f():

pass

f.x = 0

print(f.x)

| CPy output: | uPy output: |
| --- | --- |
| 0 | Traceback (most recent call last): File "<stdin>", line 10, in <module> AttributeError: 'function' object has no attribute 'x' |

Generator

Context manager __exit__() not called in a generator which does not run to completion

Sample code:

class foo(object):

def __enter__(self):

print('Enter')

def __exit__(self, *args):

print('Exit')

def bar(x):

with foo():

while True:

x += 1

yield x

def func():

g = bar(0)

for _ in range(3):

print(next(g))

func()

| CPy output: | uPy output: |
| --- | --- |
| Enter 1 2 3 Exit | Enter 1 2 3 |

Runtime

Local variables aren’t included in locals() result

Cause: MicroPython doesn’t maintain symbolic local environment, it is optimized to an array of slots. Thus, local variables can’t be accessed by a name.

Sample code:

def test():

val = 2

print(locals())

test()

| CPy output: | uPy output: |
| --- | --- |
| {'val': 2} | {'test': <function test at 0x7f3490fa0100>, '__name__': '__main__', '__file__': '<stdin>'} |

Code running in eval() function doesn’t have access to local variables

Cause: MicroPython doesn’t maintain symbolic local environment, it is optimized to an array of slots. Thus, local variables can’t be accessed by a name. Effectively, eval(expr) in MicroPython is equivalent to eval(expr, globals(), globals()).

Sample code:

val = 1

def test():

val = 2

print(val)

eval("print(val)")

test()

| CPy output: | uPy output: |
| --- | --- |
| 2 2 | 2 1 |

import

__path__ attribute of a package has a different type (single string instead of list of strings) in MicroPython

Cause: MicroPython does’t support namespace packages split across filesystem. Beyond that, MicroPython’s import system is highly optimized for minimal memory usage.

Workaround: Details of import handling is inherently implementation dependent. Don’t rely on such details in portable applications.

Sample code:

import modules

print(modules.__path__)

| CPy output: | uPy output: |
| --- | --- |
| ['/home/micropython/micropython-docs/tests/cpydiff/modules'] | ../tests/cpydiff//modules |

Failed to load modules are still registered as loaded

Cause: To make module handling more efficient, it’s not wrapped with exception handling.

Workaround: Test modules before production use; during development, use del sys.modules["name"], or just soft or hard reset the board.

Sample code:

import sys

try:

from modules import foo

except NameError as e:

print(e)

try:

from modules import foo

print('Should not get here')

except NameError as e:

print(e)

| CPy output: | uPy output: |
| --- | --- |
| foo name 'xxx' is not defined foo name 'xxx' is not defined | foo name 'xxx' isn't defined Should not get here |

MicroPython does’t support namespace packages split across filesystem.

Cause: MicroPython’s import system is highly optimized for simplicity, minimal memory usage, and minimal filesystem search overhead.

Workaround: Don’t install modules belonging to the same namespace package in different directories. For MicroPython, it’s recommended to have at most 3-component module search paths: for your current application, per-user (writable), system-wide (non-writable).

Sample code:

import sys

sys.path.append(sys.path[1] + "/modules")

sys.path.append(sys.path[1] + "/modules2")

import subpkg.foo

import subpkg.bar

print("Two modules of a split namespace package imported")

| CPy output: | uPy output: |
| --- | --- |
| Two modules of a split namespace package imported | Traceback (most recent call last): File "<stdin>", line 12, in <module> ImportError: no module named 'subpkg.bar' |

Builtin types

Generated Thu 09 Jan 2020 12:08:52 UTC

Exception

Exception chaining not implemented

Sample code:

try:

raise TypeError

except TypeError:

raise ValueError

| CPy output: | uPy output: |
| --- | --- |
| Traceback (most recent call last): File "<stdin>", line 8, in <module> TypeError During handling of the above exception, another exception occurred: Traceback (most recent call last): File "<stdin>", line 10, in <module> ValueError | Traceback (most recent call last): File "<stdin>", line 10, in <module> ValueError: |

User-defined attributes for builtin exceptions are not supported

Cause: MicroPython is highly optimized for memory usage.

Workaround: Use user-defined exception subclasses.

Sample code:

e = Exception()

e.x = 0

print(e.x)

| CPy output: | uPy output: |
| --- | --- |
| 0 | Traceback (most recent call last): File "<stdin>", line 8, in <module> AttributeError: 'Exception' object has no attribute 'x' |

Exception in while loop condition may have unexpected line number

Cause: Condition checks are optimized to happen at the end of loop body, and that line number is reported.

Sample code:

l = ["-foo", "-bar"]

i = 0

while l[i][0] == "-":

print("iter")

i += 1

| CPy output: | uPy output: |
| --- | --- |
| iter iter Traceback (most recent call last): File "<stdin>", line 10, in <module> IndexError: list index out of range | iter iter Traceback (most recent call last): File "<stdin>", line 12, in <module> IndexError: list index out of range |

Exception.__init__ method does not exist.

Cause: Subclassing native classes is not fully supported in MicroPython.

Workaround: Call using super() instead:

class A(Exception):

def __init__(self):

super().__init__()

Sample code:

class A(Exception):

def __init__(self):

Exception.__init__(self)

a = A()

| CPy output: | uPy output: |
| --- | --- |
|  | Traceback (most recent call last): File "<stdin>", line 15, in <module> File "<stdin>", line 13, in __init__ AttributeError: type object 'Exception' has no attribute '__init__' |

bytearray

Array slice assignment with unsupported RHS

Sample code:

b = bytearray(4)

b[0:1] = [1, 2]

print(b)

| CPy output: | uPy output: |
| --- | --- |
| bytearray(b'\x01\x02\x00\x00\x00') | Traceback (most recent call last): File "<stdin>", line 8, in <module> NotImplementedError: array/bytes required on right side |

bytes

bytes objects support .format() method

Cause: MicroPython strives to be a more regular implementation, so if both str and bytes support __mod__() (the % operator), it makes sense to support format() for both too. Support for __mod__ can also be compiled out, which leaves only format() for bytes formatting.

Workaround: If you are interested in CPython compatibility, don’t use .format() on bytes objects.

Sample code:

print(b'{}'.format(1))

| CPy output: | uPy output: |
| --- | --- |
| Traceback (most recent call last): File "<stdin>", line 7, in <module> AttributeError: 'bytes' object has no attribute 'format' | b'1' |

bytes() with keywords not implemented

Workaround: Pass the encoding as a positional parameter, e.g. print(bytes('abc', 'utf-8'))

Sample code:

print(bytes('abc', encoding='utf8'))

| CPy output: | uPy output: |
| --- | --- |
| b'abc' | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: keyword argument(s) not yet implemented - use normal args instead |

Bytes subscription with step != 1 not implemented

Cause: MicroPython is highly optimized for memory usage.

Workaround: Use explicit loop for this very rare operation.

Sample code:

print(b'123'[0:3:2])

| CPy output: | uPy output: |
| --- | --- |
| b'13' | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: only slices with step=1 (aka None) are supported |

dict

Dictionary keys view does not behave as a set.

Cause: Not implemented.

Workaround: Explicitly convert keys to a set before using set operations.

Sample code:

print({1:2, 3:4}.keys() & {1})

| CPy output: | uPy output: |
| --- | --- |
| {1} | Traceback (most recent call last): File "<stdin>", line 7, in <module> TypeError: unsupported types for __and__: 'dict_view', 'set' |

float

uPy and CPython outputs formats may differ

Sample code:

print('%.1g' % -9.9)

| CPy output: | uPy output: |
| --- | --- |
| -1e+01 | -10 |

int

No int conversion for int-derived types available

Workaround: Avoid subclassing builtin types unless really needed. Prefer https://en.wikipedia.org/wiki/Composition_over_inheritance .

Sample code:

class A(int):

__add__ = lambda self, other: A(int(self) + other)

a = A(42)

print(a+a)

| CPy output: | uPy output: |
| --- | --- |
| 84 | Traceback (most recent call last): File "<stdin>", line 11, in <module> File "<stdin>", line 8, in <lambda> TypeError: unsupported types for __radd__: 'int', 'int' |

list

List delete with step != 1 not implemented

Workaround: Use explicit loop for this rare operation.

Sample code:

l = [1, 2, 3, 4]

del l[0:4:2]

print(l)

| CPy output: | uPy output: |
| --- | --- |
| [2, 4] | Traceback (most recent call last): File "<stdin>", line 8, in <module> NotImplementedError: |

List slice-store with non-iterable on RHS is not implemented

Cause: RHS is restricted to be a tuple or list

Workaround: Use list(<iter>) on RHS to convert the iterable to a list

Sample code:

l = [10, 20]

l[0:1] = range(4)

print(l)

| CPy output: | uPy output: |
| --- | --- |
| [0, 1, 2, 3, 20] | Traceback (most recent call last): File "<stdin>", line 8, in <module> TypeError: object 'range' isn't a tuple or list |

List store with step != 1 not implemented

Workaround: Use explicit loop for this rare operation.

Sample code:

l = [1, 2, 3, 4]

l[0:4:2] = [5, 6]

print(l)

| CPy output: | uPy output: |
| --- | --- |
| [5, 2, 6, 4] | Traceback (most recent call last): File "<stdin>", line 8, in <module> NotImplementedError: |

str

Start/end indices such as str.endswith(s, start) not implemented

Sample code:

print('abc'.endswith('c', 1))

| CPy output: | uPy output: |
| --- | --- |
| True | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: start/end indices |

Attributes/subscr not implemented

Sample code:

print('{a[0]}'.format(a=[1, 2]))

| CPy output: | uPy output: |
| --- | --- |
| 1 | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: attributes not supported yet |

str(...) with keywords not implemented

Workaround: Input the encoding format directly. eg print(bytes('abc', 'utf-8'))

Sample code:

print(str(b'abc', encoding='utf8'))

| CPy output: | uPy output: |
| --- | --- |
| abc | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: keyword argument(s) not yet implemented - use normal args instead |

str.ljust() and str.rjust() not implemented

Cause: MicroPython is highly optimized for memory usage. Easy workarounds available.

Workaround: Instead of s.ljust(10) use "%-10s" % s, instead of s.rjust(10) use "% 10s" % s. Alternatively, "{:<10}".format(s) or "{:>10}".format(s).

Sample code:

print('abc'.ljust(10))

| CPy output: | uPy output: |
| --- | --- |
| abc | Traceback (most recent call last): File "<stdin>", line 7, in <module> AttributeError: 'str' object has no attribute 'ljust' |

None as first argument for rsplit such as str.rsplit(None, n) not implemented

Sample code:

print('a a a'.rsplit(None, 1))

| CPy output: | uPy output: |
| --- | --- |
| ['a a', 'a'] | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: rsplit(None,n) |

Instance of a subclass of str cannot be compared for equality with an instance of a str

Sample code:

class S(str):

pass

s = S('hello')

print(s == 'hello')

| CPy output: | uPy output: |
| --- | --- |
| True | False |

Subscript with step != 1 is not yet implemented

Sample code:

print('abcdefghi'[0:9:2])

| CPy output: | uPy output: |
| --- | --- |
| acegi | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: only slices with step=1 (aka None) are supported |

tuple

Tuple load with step != 1 not implemented

Sample code:

print((1, 2, 3, 4)[0:4:2])

| CPy output: | uPy output: |
| --- | --- |
| (1, 3) | Traceback (most recent call last): File "<stdin>", line 7, in <module> NotImplementedError: only slices with step=1 (aka None) are supported |

Modules

Generated Thu 09 Jan 2020 12:08:52 UTC

array

Looking for integer not implemented

Sample code:

import array

print(1 in array.array('B', b'12'))

| CPy output: | uPy output: |
| --- | --- |
| False | Traceback (most recent call last): File "<stdin>", line 8, in <module> NotImplementedError: |

Array deletion not implemented

Sample code:

import array

a = array.array('b', (1, 2, 3))

del a[1]

print(a)

| CPy output: | uPy output: |
| --- | --- |
| array('b', [1, 3]) | Traceback (most recent call last): File "<stdin>", line 9, in <module> TypeError: 'array' object doesn't support item deletion |

Subscript with step != 1 is not yet implemented

Sample code:

import array

a = array.array('b', (1, 2, 3))

print(a[3:2:2])

| CPy output: | uPy output: |
| --- | --- |
| array('b') | Traceback (most recent call last): File "<stdin>", line 9, in <module> NotImplementedError: only slices with step=1 (aka None) are supported |

builtins

Second argument to next() is not implemented

Cause: MicroPython is optimised for code space.

Workaround: Instead of val = next(it, deflt) use:

try:

val = next(it)

except StopIteration:

val = deflt

Sample code:

print(next(iter(range(0)), 42))

| CPy output: | uPy output: |
| --- | --- |
| 42 | Traceback (most recent call last): File "<stdin>", line 12, in <module> TypeError: function takes 1 positional arguments but 2 were given |

deque

Deque not implemented

Workaround: Use regular lists. micropython-lib has implementation of collections.deque.

Sample code:

import collections

D = collections.deque()

print(D)

| CPy output: | uPy output: |
| --- | --- |
| deque([]) | Traceback (most recent call last): File "<stdin>", line 8, in <module> TypeError: function missing 2 required positional arguments |

json

JSON module does not throw exception when object is not serialisable

Sample code:

import json

a = bytes(x for x in range(256))

try:

z = json.dumps(a)

x = json.loads(z)

print('Should not get here')

except TypeError:

print('TypeError')

| CPy output: | uPy output: |
| --- | --- |
| TypeError | Should not get here |

struct

Struct pack with too few args, not checked by uPy

Sample code:

import struct

try:

print(struct.pack('bb', 1))

print('Should not get here')

except:

print('struct.error')

| CPy output: | uPy output: |
| --- | --- |
| struct.error | b'\x01\x00' Should not get here |

Struct pack with too many args, not checked by uPy

Sample code:

import struct

try:

print(struct.pack('bb', 1, 2, 3))

print('Should not get here')

except:

print('struct.error')

| CPy output: | uPy output: |
| --- | --- |
| struct.error | b'\x01\x02' Should not get here |

sys

Overriding sys.stdin, sys.stdout and sys.stderr not possible

Cause: They are stored in read-only memory.

Sample code:

import sys

sys.stdin = None

print(sys.stdin)

| CPy output: | uPy output: |
| --- | --- |
| None | Traceback (most recent call last): File "<stdin>", line 8, in <module> AttributeError: 'module' object has no attribute 'stdin' |